Fort Belknap

Coordinates: 47.9672  108.7183

Nearest town: About 2 miles southwest of Hays

Elevation: 3,920 ft

Height of tower: 20 m

Height of instruments: Not documented; presumed to be standard NRG configuration

Wind vane orientation: Not documented; presumed to be true North

Logger type: NRG Wind Explorer

Quality Control

Data collection starts: 2-7-01

Data collection ends: 11-6-02

Data recovery: 91% of period

Missing data:	7-19-01 to 8-31-01

10-14-02 to 10-29-02

Observations used by DEQ: 98%

- Some periods withheld from analysis because no wind speed and/or flatline wind direction, as judged by EERC, University of North Dakota.

| Site: Fort Belknap | Height: 20 meters |  |  |  |
| --- | --- | --- | --- | --- |
| Start Date |  | End Date |  | Comment |
| 2/24/2001 |  | 2/25/2001 |  | Possible Icing Event |
| 2/8/2002 |  | 2/9/2002 |  | Possible Icing Event |
| 3/4/2002 |  | 3/8/2002 |  | Possible Icing Event |
| 3/13/2002 |  | 3/15/2002 |  | Possible Icing Event |
| 5/22/2002 |  | 5/22/2002 |  | Possible Icing Event |
| 10/29/2002 |  | 11/1/2002 |  | Possible Icing Event (Wind Direction Only) |

-The tower was part of NREL’s Native American Anemometer Loan Program.

-Data from 2-7-01 to 3-1-02 provided directly to DEQ by Tony Jimenez, NREL.  Additional data from 3-1-02 to 11-6-02 received from NREL via EERC.

-Data are presumed to be at Mountain Standard Time.