ENVIRONMENTAL ASSESSMENT

Air Quality Permitting Services Section

Air Quality Bureau

Air, Energy and Mining Division

Montana Department of Environmental Quality

Draft EA Issued: 07/29/2026

Final Decision Issued: 08/17/2026

PROJECT/SITE NAME: New Dissolved Air Flotation System-Calumet Montana Refining

APPLICANT/COMPANY NAME: Calumet Montana Refining, LLC

Montana Air Quality Permit #2161-41

LOCATION:	The facility location is 1900 10th Street Northeast Great Falls, MT 59404.

NE¼ of Section 1, Township 20 North, Range 3 East

COUNTY: Cascade

PROPERTY OWNERSHIP: 	FEDERAL:	STATE:		PRIVATE:  X

Table of Contents

Overview of Proposed Action	3

Authorizing Action	3

Description of DEQ Regulatory Oversight	3

Proposed Action	3

Purpose, Need, and Benefits	5

Other Governmental Agencies and Programs with Jurisdiction	5

Evaluation of Affected Environment And Impact by Resource	6

1.	Geology and Soil Quality, Stability and Moisture	7

2.	Water Quality, Quantity, And Distribution	8

3.	Air Quality	9

4.	Vegetation Cover, Quantity, and Quality	11

5.	Terrestrial, Avian, and Aquatic Life and Habitats	11

6.	Unique, Endangered, Fragile, or Limited Environmental Resources	12

7.	Historical and Archaeological Sites	13

8.	Aesthetics	14

9.	Demands on Environmental Resources of Land, Water, Air, or Energy	15

10.	Impacts on Other Environmental Resources	15

11.	Human Health and Safety	16

12.	Industrial, Commercial, and Agricultural Activities and Production	17

13.	Quantity and Distribution of Employment	17

14.	Local and State Tax Base and Tax Revenue	18

15.	Demand for Government Services	18

16.	Locally Adopted Environmental Plans and Goals	19

17.	Access to and Quality of Recreational and Wilderness Activities	20

18.	Density and Distribution of Population and Housing	21

19.	Social Structures and Mores	21

20.	Cultural Uniqueness and Diversity	22

21.	Private Property Impacts	23

22.	Other Appropriate Social and Economic Circumstances	23

23.	Greenhouse Gas Assessment	24

Description of Alternatives	24

Consultation	25

Public Involvement	25

Significance of Potential Impacts and Need for Further Analysis	25

Conclusions and Findings	25

Preparation	27

References	28

## Overview of Proposed Action

### Authorizing Action

Pursuant to the Montana Environmental Policy Act (MEPA), Montana agencies are required to prepare an environmental review for state actions that may have an impact on the Montana environment. The Proposed Action is a state action that may have an impact on the Montana environment; therefore, the Montana Department of Environmental Quality (DEQ) must prepare an environmental review. This EA will examine the proposed action and alternatives to the proposed action and disclose potential and proximate impacts that may result from the proposed and alternative actions. DEQ will determine the need for additional environmental review based on consideration of the criteria set forth in Administrative Rules of Montana (ARM) 17.4.608.

### Description of DEQ Regulatory Oversight

DEQ implements the Clean Air Act of Montana, overseeing the development of emitting units at Calumet Montana Refining (CMR) and associated facilities. DEQ has authority under ARM 17.8.743 to analyze the addition of a new DAF system, and administrative changes related to including final updates related to a 2002 Consent Decree.

### Proposed Action

CMR has applied for a Montana Air Quality Permit (MAQP) under the Clean Air Act of Montana, § 75-2-101, et. seq, to install a new dissolved air flotation (DAF) system which removes oil and grease from wastewater prior to discharge within the plant wastewater treatment system. Final disposition of wastewater is discharge into the Great Falls POTW. The project subject to the proposed action would be located on private land in Great Falls, Montana. All information included in this EA is derived from the permit application, discussions with the applicant, analysis of aerial photography, topographic maps, and other research tools.

Table 1. Summary of Proposed Action

| General Overview | The proposed project would add one new DAF system which would be a new emitting unit at the site. Other administrative permit changes would also be incorporated into the MAQP which are related to the 2002 Consent Decree and are necessary to allow the Consent Decree to be eligible for closure by EPA. |
| --- | --- |
| Duration & Hours of Operation | Construction: Estimated Beginning in the summer and early fall of 2026 with completion over no more than a two-month period. Construction time for this project is not extensive because the DAF system being installed was already constructed and coming to CMR as a previously used DAF system. Operation: Once complete, the DAF system would be in continuous operation. |
| Estimated Disturbance | No first-time disturbance of land will be disturbed as a result of the proposed project. |
| Construction Equipment | Construction equipment would include, but is not limited to cranes, delivery trucks, and various trade equipment such as pipe fitting and welding equipment. |
| Personnel Onsite | Construction: A small crew consisting of a group of six contractors for a 4-week construction period, managed by two CMR project managers would be used. Operation: No significant increase in permanent staff would occur. |
| Location and Analysis Area | Location: 1900 10th Street Northeast Great Falls, MT 59404 Analysis Area: The area being analyzed as part of this environmental review includes the immediate project area (Figure 1), as well as neighboring lands surrounding the analysis area, as reasonably appropriate for the impacts being considered. |

Table 2. The applicant is required to comply with all applicable local, county, state, and federal requirements pertaining to the following resource areas.

| Air Quality | The applicant proposes to add a new DAF system which would serve to strip fat and grease from the wastewater prior to discharge. Some VOC discharge increase would occur but would utilize controls including liquid absorption and activated carbon to minimize VOC release as outlined in the BACT analysis in MAQP #2161-41. |
| --- | --- |
| Water Quality | The addition of the DAF system would reduce the overall load of contaminants being discharged into the Great Falls POTW. |
| Erosion Control and Sediment Transport | The project scope would not require a large surface area of disturbance that would create concern for storm water. Contracted construction companies will be responsible for installing and maintaining erosion and sediment transport control and use Best Management Practices during the construction phase. |
| Solid Waste | Negligible to minor changes to solid waste are expected with the proposed DAF project. The DAF system would aid in internal separation of fats, oils and grease from the wastewater but facility solid waste would continue as is. |
| Cultural Resources | No impacts to cultural resources are expected with the proposed DAF project. |
| Hazardous Substances | Any hazardous substances would be handed accordingly. |
| Reclamation | No previously undisturbed land will be disturbed as a result of the proposed DAF project as the site has previously been disturbed related to its long petroleum refinery operations. |

Table 3. Cumulative Impacts

| Past Actions | Previous permitting actions have occurred related to transferring equipment from CMR to the newer Montana Renewables facility. Previous actions have addressed wastewater infrastructure but this action is occurring to provide a buffer for satisfying the Great Falls POTW Oil and Grease limit. |
| --- | --- |
| Present Actions | There are no other current actions at CMR but on June 23, 2026, an air quality permit application was received for the Montana Renewables, LLC (MRL) facility. The new MRL application would represent a fairly complex action to optimize and increase the production of sustainable aviation fuel (SAF). There likely are components of the MRL project that would change the discharge concentration from MRL operations and to the Great Falls POTW. |
| Future Related Actions | No future actions have been identified with the proposed DAF project. |

### Purpose, Need, and Benefits

DEQ's purpose in conducting this environmental review is to act upon CMR’s application for an MAQP to add a new DAF system. DEQ’s action on the permit application is governed by § 75-2-201, et seq., Montana Code Annotated (MCA) and the Administrative Rules of Montana (ARM) 17.8.740, et seq.

The applicant’s purpose and need, as expressed to DEQ in seeking this action, is to install and operate a new DAF system to operate alongside other similar infrastructure in the wastewater treatment plant.  The other requests within the application are mostly related to the planning for a consent decree closure, and are considered administrative changes.

Figure 1. Location of the Proposed  DAF Project

### Other Governmental Agencies and Programs with Jurisdiction

The proposed action would be located on private land. All applicable local, state, and federal rules must be adhered to, which may include other local, state, federal, or tribal agency jurisdiction. There are no other known agencies involved but the Great Falls POTW operations may see a negligible to minor reduction in some contaminants into their system.

## Evaluation of Affected Environment And Impact by Resource

The impact analysis will identify and evaluate the proximate direct and secondary impacts TO THE PHYSICAL ENVIRONMENT AND POPULATION IN THE AREA TO BE AFFECTED BY THE PROPOSED PROJECT. Direct impacts occur at the same time and place as the action that causes the impact. Secondary impacts are a further impact to Montana’s environment that may be stimulated, induced by, or otherwise result from a direct impact of the action (ARM 17.4.603(18)). Where impacts would occur, the impacts will be described in this analysis. When the analysis discloses environmental impacts, these are proximate impacts pursuant to 75-1-201(1)(b)(iv)(A), MCA.

Cumulative impacts are the collective impacts on Montana’s environment within the borders of Montana of the Proposed Action when considered in conjunction with other past and present actions related to the Proposed Action by location and generic type. Related future actions must also be considered when these actions are under concurrent consideration by any state agency through pre-impact statement studies, separate impact statement evaluation, or permit processing procedures (ARM 17.4.603(7)). The project identified in Table 1 was analyzed as part of the cumulative impacts assessment for each resource subject to review, pursuant to MEPA (75-1-101, et. seq).

The duration of the proposed action is quantified as follows:

- Construction Impacts (short-term): These are impacts to the environment that would occur during the construction period, including the specific range of time.

- Operation Impacts (long-term): These are impacts to the environment during the operational period of the proposed action, including the anticipated range of operational time.

The intensity of the impacts is measured using the following:

- No impact: There would be no change from current conditions.

- Negligible: An adverse or beneficial effect would occur but would be at the lowest levels of detection.

- Minor: The effect would be noticeable but would be relatively small and would not affect the function or integrity of the resource.

- Moderate: The effect would be easily identifiable and would change the function or integrity of the resource.

- Major: The effect would alter the resource.

### Geology and Soil Quality, Stability and Moisture

This section includes the following resource areas, as required in ARM 17.4.609: Geology; Soil Quality, Stability, and Moisture

Affected Environment

The proposed project would add an additional dissolved air flotation (DAF) system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going. The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

The proposed project is located within the existing property boundary of the refinery and will be confined to property currently controlled by CMR.

The affected area is primarily an industrial complex historically part of the Calumet Petroleum refinery, and in close proximity to the existing MRL renewable fuels facility which began operation in 2021. The Geology and Soil Quality, Stability and Moisture were previously evaluated under permitting actions MAQP #5263-00, MAQP #5263-01 and MAQP #5263-02. The resource area for CMR remains consistent with the earlier analyses for MRL, as the same area is being evaluated. A soils search was done specific for MAQP #5263-03 using the NRCS USDA website. The resulting survey finds that the majority of the soil classification is Kobar-Marias complex, zero to 4 percent slopes with 71 percent within this category.  The next largest classification is Marias silty clay, 2 to 4 percent slope at 16.7 percent, the third largest classification is Kobar silty clay loam, zero to 2 percent slopes at 8.8 percent.  The small soil category remaining is Kobar silty clay loam at 2 to 4 percent slopes.  Link is:

https://websoilsurvey.nrcs.usda.gov/app/WebSoilSurvey.aspx

The site is located on the north-side of the Missouri River on CMR property. adjacent to the river. The construction location for the CMR DAF equipment is located approximately 165 feet from the river’s northern edge. The elevation is approximately 3,323 feet as referenced by the nearest topographic map on the Montana DEQ GIS Layer identified as “Parcels” which has a topographic elevation marked very close to the Burlington Northern Santa Fe railway track. The ArcGIS layer link is identified as:

https://gisservicemt.gov/arcgis/rest/services/MSDI_Framework/Parcels/MapServer/0

The CMR facility is located on Pleistocene age glacial lake deposits, which overlie the consolidated Kootenai Formation. Lemke (1977) calls these sediments Deposits of Glacial Lake Great Falls. Lemke (1977) describes two subunits as an upper stratigraphic unit consisting predominantly of non-plastic fine sand and silt and a lower stratigraphic unit consisting mostly of laminated to non-laminated plastic clay and minor amounts of silt. Previous investigation activities at the CMR facility have documented the presence of unconsolidated Pleistocene fluvial and lake deposits and various fill material at the surface and immediately beneath the Site. These surficial units have been encountered at variable depths across the site that range as much as 10 to 20 ft below ground surface. The Pleistocene deposits are generally saturated but yield minimal quantities of water to wells because of their low hydraulic conductivity (Wilke 1983). (Directly from MRL – email dated 8/31/2021 from Casey Mueller).

Underlying the Pleistocene glacial lake deposits is the Cretaceous-age Kootenai formation that has been differentiated into the fifth (upper) and fourth (lower) members. The fifth member of the Kootenai formation is encountered sitewide immediately beneath the surficial Pleistocene deposits and/or fill material and is distinguished by red-weathered mudstone that contains lenses and beds of brownish-gray and greenish-gray, cross-bedded, micaceous sandstone and light gray nodular limestone concretions. The lower part contains a dark-gray shale and lignite bed with a significant pre-angiosperm flora. The bottom of the Kootenai formation’s upper member occurs at 60-100 feet below ground level near the Site. Groundwater in this unit beneath the site occurs under semiconfined conditions.

Direct Impacts

There would be no direct construction or operational impacts to geology, soil quality, stability, or moisture as a result of the project because construction-related disturbance would be negligible to minor. The current site is an already developed petroleum refinery with no first-time ground disturbances.

Secondary Impacts

There would be no secondary construction or operational impacts to geology or soil quality, stability, and moisture because construction related-disturbance would be negligible to minor. The current site is an already developed petroleum refinery with no first-time ground disturbances.

Cumulative Impacts

There would be no cumulative impacts to geology or soil quality, stability, and moisture because no direct and secondary impacts have been identified. The current site is an already developed petroleum refinery with no first-time ground disturbances.

### Water Quality, Quantity, And Distribution

This section includes the following resource areas, as required in ARM 17.4.609: Water Quality, Quantity and Distribution

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action would also provide minor permit language updates that are administrative in nature.

Direct Impacts

No direct impacts on water quality and distribution from construction-related impacts would be expected because minimal water use would be expected for construction activities. A minor reduction in contaminants in the form of fats and grease would be expected because the DAF system would remove contaminants prior to discharge into the Great Falls POTW.  The specific reduction target was not provided within the application but the function of the DAF system is to allow a better separation of materials within the wastewater and to meet a 100 mg/l fat and grease POTW limit. The additional DAF would provide a buffer for conducting maintenance activities which would impact oil and grease levels in the wastewater.

Secondary Impacts

A secondary impact of slightly reduced load to the existing Great Falls POTW is likely because the DAF system would reduce contaminants reaching the POTW. This reduction may provide a larger buffer for ensuring the oil and grease discharge limit is achieved.

Cumulative Impacts

A possible, potential cumulative impact would be delaying possible required POTW upgrades because the DAF system may marginally reduce the current CMR discharge, which utilizes a portion of the POTW capacity. Previously, it was established that a Montana Renewables permit application was received and is now in process, and the MRL project may impact the BOD/COD discharge to the Great Falls POTW.

### Air Quality

This section includes the following resource areas, as required in ARM 17.4.609: Air Quality

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. Scenario #3 represents the highest level of potential emissions. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

The proposed project is located within the existing property boundary of the refinery and will be confined to property currently controlled by CMR.

The affected area is unclassifiable or attainment for all pollutants with an applicable NAAQS. Existing sources of air pollution in the area include emissions from the Montana Renewables, LLC (MRL) site.  Under the recent MAQP issuance for the MRL facility (December 31, 2025), a full impact analysis was conducted for PM2.5 and NO2 for the MRL facility including nearby sources and background concentrations and found no indication of NAAQS violations. That analysis can be found on the DEQ website for MAQP #5263-03.  This DAF project would increase VOC emissions by up to 36 tons per year if the highest emitting scenario is selected.

Applicants are required to comply with all laws relating to air, such as the Federal Clean Air Act, NAAQS set by the Environmental Protection Agency (EPA), and the Clean Air Act of Montana.

In addition, MAQP #2161-41 provides legally enforceable conditions regarding the emitting units themselves, pollution controls, and requires the applicant to take reasonable precautions to limit fugitive dust from this location.

The emission inventory, located in Section IV of the MAQP Analysis estimates new potential emissions associated with the proposed addition of the new DAF system.

Direct Impacts

Emissions resulting from the permit action would be considered minor with the largest VOC increase for the five scenarios being 36.2 tpy occurring for Scenario 3. This calculation is based on the potential to emit increase for the new DAF above the current actual 2-year baseline (2024-2025).  Scenario #3 would result in total VOC emissions increasing to 67.4 tpy including the baseline. The only pollutant increase that would occur would be for VOCs. The proposed VOC increase would be less than the significant emission rate (40 tpy) that would result in the project being considered major for Prevention of Significant Deterioration (PSD). Operational direct impacts would be long-term, and consistent with existing impacts, and mitigated by implementation of enforceable limits and conditions including controls established by the BACT analysis. The BACT conditions are located within the MAQP document.

Potential Emission Resulting Totals and Increases for the DAF Options Evaluated

| Emission Unit Name/Description | Criteria Pollutant (tons/year) |
| --- | --- |
|  | VOC |
| WW Separator (Scenario 1) | 42.61 |
| WW Separator (Scenario 2) | 43.29 |
| WW Separator (Scenario 3)3 | 67.40 |
| WW Separator (Scenario 4) | 40.71 |
| WW Separator (Scenario 5) | 41.35 |
| Current WW Separator EU16 Baseline Emissions1 | 31.25 |
| Project Emission Increase (due to new DAF- Scenario 3 shown as highest emitting scenario) | 36.15 |
| Significant Emission Rate (SER) Threshold2 | 40 |
| Is the Project Emission Increase > SER? | No |

MRL previously demonstrated compliance with applicable NAAQS and MAAQS standards (PM2.5, NO2, CO and ozone) and the CMR VOC increase would not likely impact ambient air because of the potential VOC increase.

Adverse air quality impacts would be minor because of the proposed project. See the permit analysis for more information regarding air quality impacts.  The majority of pollutants from the proposed project would be related to air stripping of VOCs from wastewater in the new DAF. Because increased emissions would be minimal and appropriately controlled pursuant to the current permit action any impacts would be minor.

Secondary Impacts

Emissions from the proposed project would use the established BACT controls  which have effectively been used on similar processes involving VOCs from wastewater. Secondary NAAQS provides public welfare protection, including protection against decreased visibility and damage to animals, crops, vegetation, and buildings.  See permit analysis for more detailed information regarding air quality impacts. Any adverse impacts would be long-term and minor. No beneficial secondary impacts would be expected because of the proposed project.

Cumulative Impacts

Cumulative impacts from the proposed DAF project would be restricted by conditions and limits contained in the MAQP; therefore, any expected air quality impacts would be minor. The Cascade County area also has the adjacent MRL renewable fuels plant and other nearby stationary sources are generally minor sources of air emissions. The cumulative impacts of these other emitters and the proposed action would not have an adverse impact to air quality. Impacts from the Permitting Action are limited by enforceable conditions and limits contained in the MAQP and BACT limits established.

Because emissions from the proposed project, and all other similar or related projects located in the affected area are regulated, any adverse cumulative impacts to air quality would be long-term and minor due to the continued operation of the refinery.

### Vegetation Cover, Quantity, and Quality

This section includes the following resource areas, as required in ARM 17.4.609: Vegetation Cover, Quantity and Quality

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system. CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts

No direct construction or operational impacts to vegetative cover, quantity, or quality would be expected as a result of the proposed action because there are no first-time areas of disturbance associated with the proposed action.

Secondary Impacts

No secondary construction or operational impacts to vegetative cover, quantity, or quality would be expected as a result of the proposed action because there are no first-time areas of disturbance associated with the proposed action.

Cumulative Impacts

There will be no cumulative impacts to vegetative cover, quantity, or quality associated with the proposed action because no direct and secondary impacts have been identified.

### Terrestrial, Avian, and Aquatic Life and Habitats

This section includes the following resource areas, as required in ARM 17.4.609: Terrestrial and Aquatic Life and Habitats; Unique, Endangered, Fragile, or Limited Environmental Resources

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts

No direct construction or operational impacts to terrestrial, avian and aquatic life and habitats would be expected as a result of the proposed action because there are no first-time areas of disturbance associated with the proposed action

Secondary Impacts

No secondary construction or operational impacts to terrestrial, avian and aquatic life and habitats would be expected as a result of the proposed action because there are no first-time areas of disturbance associated with the proposed action.

Cumulative Impacts

There will be no cumulative impacts to terrestrial, avian and aquatic life and habitats associated with the proposed action because no direct and secondary impacts have been identified.

### Unique, Endangered, Fragile, or Limited Environmental Resources

This section includes the following resource areas, as required in ARM 17.4.609: Unique, Endangered, Fragile, or Limited Environmental Resources.

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

The proposed project is not in core, general or connectivity sage grouse habitat, as designated by the Sage Grouse Habitat Conservation Program at: http://sagegrouse.mt.gov.

A search of the Montana Natural Heritage Program in the affected area identified the following potential species occurring in the area.

DEQ earlier conducted a search using the Montana Natural Heritage Program (MTNHP) site for the original 44.46 acre parcel. A MTNHP Environmental Summary report was downloaded on October 9, 2025 for a previous action at the CMR/MRL site. The default polygons for the selected area includes area into and across the Missouri River with the default polygons totaling approximately two square miles (1280 acres). The MTNHP surveys cover the entire CMR and MRL parcels and beyond.

Species of concern (SOC) from the new MTNHP report identified the following species: Great Blue Heron, Common Tern, Spiny softshell, Forster’s Tern, American White Pelican, Black-crowned Night Heron, White-faced ibis, Caspian Tern, Barrow’s Goldeneye, Black-necked Stilt, Sharp-tailed Grouse, Black Tern, Ferruginous Hawk, Horned Grebe, American Goshawk, Cassin’s Finch, Brewer’s Sparrow, Franklin’s Gull, Clark’s Grebe, common Loon, Solitary Sandpiper, Trumpeter Swan, and Harlequin Duck. Many of these species listed as SOC have not been observed within the search polygon. The one exception noted is that Bald Eagles have been observed.

Direct Impacts

The majority of species of concern from the MTNHP list are associated with the riverine habitat on the Missouri River, which is approximately 165 feet to the south of the proposed action. These species would not be displaced by the proposed action as the site is completely industrial and the parcel in question does not contact the river or river banks. The potential impact (including cumulative impacts) to species present including bald eagles would be negligible.

.

Secondary Impacts

No secondary impacts from construction or operations are expected as a result of the proposed project. The affected area is an already developed petroleum refinery with no terrestrial, avian, or aquatic habitats located within the CMR property boundary.

Because the area surrounding the proposed DAF project area is an already developed industrial complex, no species are expected to be present.

Cumulative Impacts

There will be no cumulative impacts to unique, endangered, fragile or limited environmental resources associated with the proposed action based on no direct and secondary impacts having been identified.

### Historical and Archaeological Sites

This section includes the following resource areas, as required in ARM 17.4.609: Historical and Archaeological Sites

Affected Environment

The Montana State Historic Preservation Office (SHPO) was notified of a previous application for the original MAQP #5263-00, occurring at the same industrial CMR/MRL site. A new search was not conducted for this application given it is at the same property location. SHPO conducted a file search and provided a letter dated August 25, 2021. The SHPO search was conducted for Section 1 T20N R3E. This proposed project does not occur outside the original search area, and is not proposed to disturb ground which has not been previously disturbed.  However, the original SHPO findings for the project area are included below.

The file search identified 19 cultural resource sites within the search area criteria.

It is SHPO’s position that any structure over fifty years of age are considered historic and are potentially eligible for listing on the National Register of Historic Places. If any structures are within the Area of Potential Effect, and are over fifty years old, SHPO recommends that they be recorded, and a determination of their eligibility be made prior to any disturbance taking place.

Direct Impacts

No direct construction or operational impacts to historical or archaeological sites would be expected as a result of the proposed action because the proposed DAF project would occur in an already existing, industrial area within the CMR property boundary.

Secondary Impacts

No secondary construction or operational impacts to historical or archaeological sites would be expected as a result of the proposed action because there would be no new ground disturbance.

Cumulative Impacts

There would be no cumulative impacts to historical or archaeological sites associated with the proposed action based on direct and secondary impacts.

### Aesthetics

This section includes the following resource areas, as required in ARM 17.4.609: Aesthetics

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts

No direct construction or operational impacts to the aesthetics would be expected as a result of the proposed action because the proposed DAF project would occur in an already existing, industrial area within the CMR property boundary. The DAF construction area would be approximately 165 feet from the northern edge of the Missouri River. No equipment additions would be expected to add additional height or noise to the existing facility.

Secondary Impacts

No secondary construction or operational impacts to the aesthetics are expected as a result of the proposed action because the proposed DAF project would occur in an already existing, industrial area within the CMR property boundary.

Cumulative Impacts

There will be no cumulative impacts to aesthetics associated with the proposed action based on no direct and secondary impacts having been identified.

### Demands on Environmental Resources of Land, Water, Air, or Energy

This section includes the following resource areas, as required in ARM 17.4.609: Demands on Environmental Resources of Land, Water, Air, or Energy

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Physical changes to the facility would accommodate the additional DAF system. The new and modified equipment would contribute to additional equipment, potentially with additional resource demands of land, water, and energy.

Direct Impacts

During construction of the proposed action there would be minor increase of energy use to construct the proposed action. Once operational, energy and electric demands would continue for the duration of the facility’s lifetime.

The proposed action would increase the facility demands for energy which would be considered minor and long-term.

Secondary Impacts

If the contaminant load discharged to the Great Falls POTW drops as a result of the new DAF, a negligible to minor beneficial impact could occur for energy use at the Great Falls POTW.

Cumulative Impacts

There would be no cumulative impacts to the demands of environmental resources of land, water, or air.

### Impacts on Other Environmental Resources

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Other Environmental Resources

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts

No direct construction or operational impacts on demands of environmental resources would be expected as a result of the proposed action because the proposed action is located inside of an existing facility and would not affect any outside environmental resources.

Secondary Impacts

No secondary construction or operational impacts demands of environmental resources would be expected as a result of the proposed action because the proposed action is located inside of an existing facility and would not affect any outside environmental resources.

Cumulative Impacts

No other impacts to environmental resources, beyond the resource areas already covered within this EA would result in any known additional cumulative impacts based on direct and secondary impacts.

### Human Health and Safety

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Human Health and Safety

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

The applicant would be required to adhere to all applicable state and federal safety laws. The Occupational Safety and Health Administration (OSHA) has developed rules and guidelines to reduce the risks associated with this type of labor. Few, if any, members of the public would be in immediate proximity to the project during construction or operations.

Direct Impacts:

No direct construction or operational impacts to human health and safety would be expected as a result of the proposed action. Emissions released into the human environment from the facility due to the proposed action would be considered minor.

Secondary Impacts:

No secondary construction or operational impacts to human health and safety are expected as a result of the proposed action.

Cumulative Impacts:

No other affects to human health and safety, beyond the resource areas already covered within this EA would result in any known additional cumulative impacts.

### Industrial, Commercial, and Agricultural Activities and Production

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Human Health and Safety

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

The applicant would be required to adhere to all applicable state and federal safety laws. The Occupational Safety and Health Administration (OSHA) has developed rules and guidelines to reduce the risks associated with this type of labor.

Direct Impacts

No direct construction or operational impacts to commercial or agricultural activities are expected. Negligible long-term impacts to industrial production would be expected as a result of the proposed DAF project because refinery capacity is not expanding for products.

Secondary Impacts

No secondary construction impacts to industrial, commercial, or agricultural activities are expected with the proposed project.

Cumulative Impacts

No other environmental resources, beyond the resource areas already covered within this EA would result in any known additional cumulative impacts based on direct and secondary impacts.

### Quantity and Distribution of Employment

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Quantity and Distribution of Employment

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts

CMR would use existing staff to operate the proposed DAF system with operations likely being automatic. Therefore, any direct impacts to the quantity and distribution of employment in the affected area would be short-term and negligible, limited to the construction staff used for the project.

Secondary Impacts

CMR would use existing staff to operate the proposed DAF system with operations likely being automatic. Therefore, any secondary impacts to the quantity and distribution of employment in the affected area would be long-term and negligible.

Cumulative Impacts

No cumulative impacts would be expected on long-term employment from the proposed action because the facility would not be expected to create any permanent new jobs.

### Local and State Tax Base and Tax Revenue

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Local and State Tax Base and Tax Revenue

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts

A negligible to minor beneficial increase from construction labor to the local and state tax base and tax revenue would be expected as a result of the proposed action.

Secondary Impacts

If the contaminant load drops to the Great Falls POTW, a fee decrease may occur because POTW charges are often based on flow and concentration of incoming effluent. Therefore, a negligible to minor long-term economic benefit to CMR may occur, and an equal negligible to minor long-term loss of revenue could occur for the Great Falls POTW. Local, state and federal governments would be responsible for appraising the property, setting tax rates, collecting taxes, from the companies, employees, or agricultural landowners benefitting from the proposed operation. Further, MRL would be responsible for accommodation of any increased taxes associated with operation of the proposed facility. Therefore, any secondary impacts would be negligible to minor, consistent with existing impacts in the affected area, and beneficial. No adverse secondary impacts would be expected because of the proposed project.

Cumulative Impacts

Long-term beneficial negligible minor impacts to local and state tax base and tax revenues are anticipated from this permitting action due to construction labor for the project.

### Demand for Government Services

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Demands for Government Services

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts:

The air quality permit has been prepared by state government employees as part of their day-to-day, regular responsibilities. Therefore, any adverse direct impacts to demands for government services is consistent with existing impacts and negligible. No beneficial direct impacts would be expected because of the proposed project.

Secondary Impacts:

Ongoing compliance inspections of facility operations would be accomplished by state government employees as part of their typical, regular duties and required to ensure the facility is operating within the limits and conditions listed in the air quality permit. Therefore, any adverse secondary impacts to demands for government services would be consistent with existing impacts and negligible. No beneficial secondary impacts would be expected because of the proposed project.

Cumulative Impacts:

Minor cumulative impacts are anticipated on government services with the proposed action because regulators would be responsible for inspecting the proposed changes to the existing facility. However, any impacts would be negligible and consistent with existing impacts because regulators would likely combine visits to cover regulatory oversight needs.

### Locally Adopted Environmental Plans and Goals

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Locally Adopted Environmental Plans and Goals

Affected Environment

A review was conducted of the City of Great Falls website on August 18, 2025, for MAQP #5263-03. This CMR action is occurring at the same site. A zoning map was previously reviewed for earlier permitting actions, and the proposed project would be located on an I-2 Heavy Industrial Zoned parcel.  Other Planning documents were also viewed one of which was a Missouri River Urban Corridor Plan (Plan). The CMR property near the Missouri River is unlikely to be an area where the preservation of river frontage is addressed by the Plan. The website indicates that the City is updating their Growth Plan Policy indicating it will include a 2024-2025 update.

The below information was taken from the Environmental Division from the City of Great Falls website and is included for additional reference.

The Environmental Division website indicates their objectives as:

- Protect and implement water quality standards.

- Oversee and implement the Industrial Pretreatment Program (IPT).

- Manage a Fats, Oil and Grease (FOG) and Trucked and Hauled Waste Sector Control Programs.

- Manage and implement the City of Great Falls Municipal Separate Storm Sewer System (MS4) Program.

- Educate the general public and consultants on environmental regulations.

- Respond to the public's environmental needs

Direct Impacts:

No locally adopted environmental plans and goals were identified other than those listed above as objectives for the Environmental Division of the City of Great Falls. CMR would be responsible for any of their operations which are covered by the above objectives.  Specifically, CMR would continue to haul raw materials which may be subject to the fats, oil and grease program, and likely also subject to stormwater and industrial pretreatment programs.  These programs would be outside the regulatory authority established in the Air Quality Permit.

Secondary Impacts:

While some planning objectives are established for the City of Great Falls, these objectives would be expected to get included as part of the normal procedures of the City of Great Falls working with area industrial businesses. Negligible to minor secondary impacts to locally adopted environmental plans and goals would be expected because of the proposed project.

Cumulative Impacts:

No cumulative impacts to the locally adopted environmental plans and goals are anticipated since no direct impacts or secondary impacts were identified.

### Access to and Quality of Recreational and Wilderness Activities

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Access to and Quality of Recreation and Wilderness Activities

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system. CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Recreation opportunities are located to the south of the proposed action via water-activities on the Missouri River. No wilderness areas or other recreational sites are in the vicinity.

Direct Impacts:

There are no wilderness areas that occur in the vicinity of the proposed project. There would be no impacts to wilderness access as none are in the vicinity of the proposed action. Recreationalists on the Missouri River would unlikely be able to see much of the changes related to the project. The noise would not be expected to add to the existing baseline levels. Duration would be expected to be long-term and negligible.

Secondary Impacts:

The effected area consists primarily of industrial property. The project would have no impacts on the immediate area, therefore, no secondary impacts to access and quality of recreational and wilderness activities would be expected because of proposed facility operations..

Cumulative Impacts:

No cumulative impacts to access and quality of recreational and wilderness activities are anticipated as a result of the proposed permitting action based on direct and secondary impacts.

### Density and Distribution of Population and Housing

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Density and Distribution of Population and Housing

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts:

CMR would employ existing staff to operate the new DAF system and would not be expected to increase or decrease the local population outside of normal employee turnover. Therefore, no direct impacts to density and distribution of population and housing would be expected because of the proposed project. Construction employees would be temporary and would not likely require additional housing.

Secondary Impacts:

CMR would employ existing staff to operate the new DAF, and the proposed project would not be expected to otherwise result in an increase or decrease in the local population. Therefore, no secondary impacts to density and distribution of population and housing would be expected because of the proposed project.

Cumulative Impacts:

No cumulative impacts to density and distribution of population and housing are anticipated as a result of the proposed permitting. There are no impacts on the density and distribution of population and housing.

### Social Structures and Mores

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Social Structures and Mores

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

DEQ is not aware of any Native American cultural concerns that would be aﬀected by the proposed action. Based on the information provided by the Applicant, it is not anticipated that this project would disrupt traditional lifestyles or communities.

The existing nature of the area affected by the proposed project is industrial.

Direct Impacts:

Construction and operation of the new DAF would not be expected to affect the existing customs and values of the affected population. Therefore, no direct impacts to the existing social structures and mores of the affected population would be expected because of the proposed project.

Secondary Impacts:

The existing nature of the area affected by the proposed project is industrial (petroleum refining) therefore, operation of the facility would not be expected to affect the existing customs and values of the affected population. Therefore, no secondary impacts to the existing social structures and mores of the affected population would be expected because of the proposed project.

Cumulative Impacts:

No cumulative impacts to social structures and mores are anticipated as a result of the proposed permitting. There would be no impacts on the social structures and mores.

### Cultural Uniqueness and Diversity

This section includes the following resource areas, as required in ARM 17.4.609: Impacts to Cultural Uniqueness and Diversity

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

The existing nature of the area affected by the proposed project is industrial (petroleum refining). It is not anticipated that this project would cause a shift in some unique quality of the area.

Direct Impacts:

CMR would employ existing staff to operate the new DAF system and would not be expected to result in an increase or decrease in the local population. Therefore, no direct impacts to the existing cultural uniqueness and diversity of the affected population would be expected because of the proposed action.

Secondary Impacts:

The existing nature of the area affected by the proposed project is industrial (petroleum refining). Further, CMR would employ existing staff to operate the new DAF system and thus the proposed project would not be expected to result in an increase or decrease in the local population.

Therefore, no secondary impacts to the existing cultural uniqueness and diversity of the affected population are anticipated as a result of the proposed action.

Cumulative Impacts:

No cumulative impacts to cultural uniqueness and diversity are anticipated because the skills required by this project would be similar to current job requirements completed with existing staff..

### Private Property Impacts

The proposed project would take place on private land owned by the applicant. DEQ’s approval of MAQP #2161-41 would affect the applicant’s real property. DEQ has determined, however, that the permit conditions are reasonably necessary to ensure compliance with applicable requirements under the CAA Act. Therefore, DEQ’s approval of MAQP #2161-41 would not have private property-taking or damaging implications

### Other Appropriate Social and Economic Circumstances

This section includes the following resource areas, as required in ARM 17.4.609: Impacts to Other Appropriate Social and Economic Circumstances

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

Direct Impacts:

DEQ is unaware of any other appropriate short-term social and economic circumstances in the affected area that may be directly impacted by the proposed project. Due to the nature of the proposed action, no further direct impacts would be expected because of the proposed project.

Secondary Impacts:

DEQ is unaware of any other appropriate long-term social and economic circumstances in the affected area that may be impacted by the proposed project. No further secondary impacts would be expected because of the proposed project.

Cumulative Impacts:

No cumulative impacts to any other appropriate social and economic circumstances are anticipated because no direct and secondary impacts were identified.

### Greenhouse Gas Assessment

Affected Environment

The proposed DAF project would add an additional DAF system to the existing wastewater system.  CMR has proposed five possible scenarios that are being evaluated, with engineering design and evaluation on-going.  The permit would provide the flexibility for CMR to choose any of the five choices, and this analysis considers the highest level of emissions possible from the five scenarios. The scenarios consider how the new system would function with the existing DAF systems that are already in place at the site. The action also would provide minor permit language updates that are administrative in nature.

For the purpose of this analysis, DEQ has defined greenhouse gas emissions as the following gas species: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), and many species of fluorinated compounds.

The range of fluorinated compounds includes numerous chemicals which are used in many household and industrial products. Other pollutants can have some properties that also are similar to those mentioned above, but the EPA has clearly identified the species above as the primary GHGs.  Water vapor is also technically a greenhouse gas, but its properties are controlled by the temperature and pressure within the atmosphere, and it is not considered an anthropogenic species.

The project design for the DAF system incorporates two levels of control that serve to prevent any release of VOCs that would contribute to GHGs.  The first level of control uses liquid absorption to trap and collect the majority of VOCs within liquid diesel fuel which is then returned into the process for further process handling.  Residual VOCs that are not absorbed into the diesel fuel are then routed through an activated carbon device preventing VOC release to the environment. Further, the nature of VOC emissions from wastewater is not characterized by VOC species which are dominated by GHGs.

This action for a DAF has no GHG impacts and no further discussion of GHGs is presented.

### Description of Alternatives

No Action Alternative: In addition to the proposed action, DEQ must also consider a "no action" alternative. The "no action" alternative would deny the approval of the new DAF. The applicant would lack the authority to conduct the proposed activity. Any potential impacts that would result from the proposed action would not occur. The “no action” alternative forms the baseline from which the impacts of the proposed action can be measured.

If the applicant demonstrates compliance with all applicable rules and regulations required for approval, the “no action” alternative would not be appropriate.

Other Reasonable Alternative(s): No reasonable alternatives were identified for the proposed action.

### Consultation

DEQ engaged in internal and external efforts to identify substantive issues and/or concerns related to the proposed project. Internal scoping consisted of internal review of the environmental assessment document by DEQ staff. External scoping efforts also included queries to the following websites/databases/personnel.

A review of the Cascade County website, and listed department information identified several programs such as Cascade County Zoning Regulations (Revised December 2021), Cascade County Floodplain Regulations (Revised March 2013), Cascade County Subdivisions (Revised 2018) and a Growth Policy Update May 2014. There may be elements of these programs that would apply to land development, industry standards and manufacturing that might be covered by these over-arching policies.

### Public Involvement

The public comment period for this permit action will occur from 7/29/2026 through 8/13/2026.

### Significance of Potential Impacts and Need for Further Analysis

When determining whether the preparation of an environmental impact statement is needed, DEQ is required to consider the seven significance criteria set forth in ARM 17.4.608, which are as follows:

- The severity, duration, geographic extent, and frequency of the occurrence of the impact;

- The probability that the impact will occur if the proposed action occurs; or conversely, reasonable assurance in keeping with the potential severity of an impact that the impact will not occur;

- Growth-inducing or growth-inhibiting aspects of the impact, including the relationship or contribution of the impact to cumulative impacts – identify the parameters of the proposed action;

- The quantity and quality of each environmental resource or value that would be affected, including the uniqueness and fragility of those resources and values;

- The importance to the state and to society of each environmental resource or value that would be affected;

- Any precedent that would be set as a result of an impact of the proposed action that would commit the department to future actions with significant impacts or a decision in principle about such future actions; and

- Potential conflict with local, state, or federal laws, requirements, or formal plans.

### Conclusions and Findings

DEQ finds that this action results in negligible to minor impacts to air quality and GHG emissions in Cascade County, Montana.

No significant adverse impacts would be expected because of the proposed project. As noted through the draft EA, the severity, duration, geographic extent and frequency of the occurrence of the impacts associated with the proposed air quality project would be limited. The proposed action adds a new DAF system to the existing wastewater treatment system.

The proposed DAF project would improve the removal of oils, fats and grease to achieve a better separation of materials from the wastewater.

As discussed in this EA, DEQ has not identified any signiﬁcant impacts associated with the proposed actions for any environmental resource. DEQ does not believe that the activities proposed by the Applicant would have any growth-inducing or growth-inhibiting aspects, or contribution to cumulative impacts.

There are no unique or known endangered fragile resources in the project area and no first-time ground disturbance would be required for this project.

There would be no impacts to view-shed aesthetics as the site is an already existing refinery.

Demands on the environmental resources of land, water, air, or energy would not be signiﬁcant.

Impacts to human health and safety would not be signiﬁcant as entry points would be closed to the public and because the site is on private land.

As discussed in this EA, DEQ has not identified any signiﬁcant impacts associated with the proposed activities on any environmental resource.

Issuance of a Montana Air Quality Permit #2161-41 to the Applicant does not set any precedent that commits DEQ to future actions with signiﬁcant impacts or a decision in principle about such future actions. If the Applicant submits another modification or proposes to amend the permit, DEQ is not committed to issuing those revisions.

DEQ would conduct an environmental review for any subsequent permit modifications sought by the Applicant pursuant to MEPA. DEQ would make permitting decisions based on the criteria set forth in the Clean Air Act of Montana.

Issuance of the Permit to the Applicant does not set a precedent for DEQ’s review of other applications for Permits, including the level of environmental review. The level of environmental review decision is made based on case-speciﬁc consideration of the criteria set forth in ARM 17.4.608.

Finally, DEQ does not believe that the proposed air quality permitting action by the Applicant would have any growth-inducing or growth inhibiting impacts that would conﬂict with any local, state, or federal laws, requirements, or formal plans.

Based on a consideration of the criteria set forth in ARM 17.4.608, no significant adverse impacts to the affected human environment would be expected because of the proposed project. Therefore, preparation of an Environmental Impact Statement or EIS is not required, and the draft EA is deemed the appropriate level of environmental review pursuant to MEPA.

## Preparation

Environmental Assessment and Significance Determination Prepared By:

Craig Henrikson, P.E.

Environmental Engineer

Approved By:

July 24, 2026

Eric Merchant, Supervisor			     Date

Air Quality Permitting Services Section

Air Quality Bureau

Air, Energy and Mining Division

Montana Department of Environmental Quality

## References

- Air Quality Permit Application Draft Received 6/10/2026

- Air Quality Permit Signed Application Received 6/18/2026

- Correspondence Received 6/16/2026 from Ramboll on behalf of CMR

- Correspondence received 6/24/2026 on Environmental Assessment Questions

- Correspondence received 7/17/2026 from Ramboll on BACT and Federal Subpart applicability

- Relied upon previous MAQP permit issuances (and associated EAs) for CMR (2161) and MRL (5263) related to the Environmental Assessment specifically for soils, SHPO and NRIS as they are on the same geographic properties.

- 2021 BLM Specialist Report on Annual Greenhouse Gas Emissions and Climate Trends, https://www.blm.gov/

- 2022 BLM Specialist Report on Annual Greenhouse Gas Emissions and Climate Trends, BLM Greenhouse Gas Emissions and Climate Trends

- BLM 2023

2023 BLM Specialist Report:

2023 BLM Specialist Report on Annual Greenhouse Gas Emissions and Climate Trends

Montana State Historical Preservation Office (SHPO) Report from August 25, 2021

Montana Natural Heritage Program (Website Search Downloads) From Download Aug 15, 2021

Montana Natural Heritage Program Download October 9, 2025

Montana DEQ Cadastral GIS Layer – Through-Out Project Up Until Draft Issuance

NRCS Soil Survey Website Accessed October 6, 2025

City of Great Falls Website – Planning Documents

Cascade County Website

Wind Rose Information – Great Falls International Airport