ENVIRONMENTAL ASSESSMENT

July 30, 2026

Air Quality Permitting Services Section

Air Quality Bureau

Air, Energy and Mining Division

Montana Department of Environmental Quality

Project/Site Name: Bear Paw Compressor Station

Applicant Name: NorthWestern Energy

Action: Montana Air Quality Permit #5358-00

Location: NE ¼ of the SE ¼ in Section 7, Township 27N Range 19E in Blaine County, Montana or lat/long 48.10877, -109.31952

County: Blaine

Property Ownership:

FEDERAL ☐

STATE ☐

PRIVATE ☒

Draft EA: 07/13/2026

Final Decision EA: 07/30/2026

EA:

Table of Contents

Overview of Proposed Action	3

Authorizing Action	3

Description of DEQ Regulatory Oversight	3

Proposed Action	3

Purpose, Need, and Benefits	5

Other Governmental Agencies and Programs with Jurisdiction	6

Evaluation of Affected Environment And Impact by Resource	7

1.	Geology and Soil Quality, Stability and Moisture	8

2.	Water Quality, Quantity, And Distribution	8

3.	Air Quality	9

4.	Vegetation Cover, Quantity, and Quality	11

5.	Terrestrial, Avian, and Aquatic Life and Habitats	12

6.	Unique, Endangered, Fragile, or Limited Environmental Resources	14

7.	Historical and Archaeological Sites	15

8.	Aesthetics	15

9.	Demands on Environmental Resources of Land, Water, Air, or Energy	16

10.	Impacts on Other Environmental Resources	17

11.	Human Health and Safety	18

12.	Industrial, Commercial, and Agricultural Activities and Production	18

13.	Quantity and Distribution of Employment	19

14.	Local and State Tax Base and Tax Revenue	20

15.	Demand for Government Services	20

16.	Locally Adopted Environmental Plans and Goals	21

17.	Access to and Quality of Recreational and Wilderness Activities	21

18.	Density and Distribution of Population and Housing	22

19.	Social Structures and Mores	22

20.	Cultural Uniqueness and Diversity	23

21.	Private Property Impacts	23

22.	Other Appropriate Social and Economic Circumstances	24

23.	Greenhouse Gas Assessment	24

Description of Alternatives	26

Consultation	26

Public Involvement	26

Significance of Potential Impacts and Need for Further Analysis	26

Conclusions and Findings	27

Preparation	29

References	30

## Overview of Proposed Action

### Authorizing Action

Pursuant to the Montana Environmental Policy Act (MEPA), Montana agencies are required to prepare an environmental review for state actions that may have an impact on the Montana environment. The Proposed Action is a state action that may have an impact on the Montana environment; therefore, the Montana Department of Environmental Quality (DEQ) must prepare an environmental review. This EA will examine the proposed action and alternatives to the proposed action and disclose potential and proximate impacts that may result from the proposed and alternative actions. DEQ will determine the need for additional environmental review based on consideration of the criteria set forth in Administrative Rules of Montana (ARM) 17.4.608.

### Description of DEQ Regulatory Oversight

DEQ implements the Clean Air Act of Montana, overseeing the development of a natural gas pipeline and compressor station and associated facilities. DEQ has authority to analyze proposed emitting units subject to rule established in ARM 17.8.743

### Proposed Action

NorthWestern Energy (NWE) has applied for a Montana Air Quality Permit (MAQP) under the Clean Air Act of Montana, § 75-2-101, et. seq, to construct a natural gas pipeline compressor station. The project is subject to the proposed action would be located on private land, in Blaine County, Montana. All information included in this EA is derived from the permit application, discussions with the applicant, analysis of aerial photography, topographic maps, and other research tools.

Table 1. Summary of Proposed Action

| General Overview | The proposed action would allow NWE to construct and operate a natural gas line compressor station that boosts natural gas pressure using two (2) Waukesha L7044GSI S5 AFR3 1900 – 1900 brake horsepower (bhp) engines along with a 261 bhp emergency backup generator. |
| --- | --- |
| Duration & Hours of Operation | Construction: Construction for the facility is expected to be completed in less than 3 months. Operation: The facility would operate 8760 hours per year (continuous operation) |
| Estimated Disturbance | The disturbances associated with this permit would be negligible and take place on a previously permitted oil and gas infrastructure site. |
| Construction Equipment | Heavy construction equipment would only be present for the construction period, approximately 3 months, while construction is performed. |
| Personnel Onsite | Construction: contracted Operation: The site generally is operated remotely with no on-site staff. |
| Location and Analysis Area | Location: NE ¼ of the SE ¼ in Section 7, Township 27N Range 19E in Blaine County, Montana or lat/long 48.1083 N/-109.3184 W Analysis Area: The area being analyzed as part of this environmental review includes the immediate project area (Figure 1), as well as neighboring lands surrounding the analysis area, as reasonably appropriate for the impacts being considered. |

Table 2. The applicant is required to comply with all applicable local, county, state, and federal requirements pertaining to the following resource areas.

| Air Quality | The applicant proposes to install and operate two (2) 1900 bhp natural gas fired engines to compress natural gas, boosting pressure on a natural gas pipeline. MAQP #5358-00 incorporates conditions and limitations that would protect air quality within the operational site and surrounding area. |
| --- | --- |
| Water Quality | The applicant proposes installing and operating a natural gas pipeline compressor station fueled by natural gas. No water would be required for the operation of the engines; therefore, water quality would not likely be impacted as a result of the proposed action. |
| Erosion Control and Sediment Transport | The applicant proposes to install and operate a natural gas pipeline compressor station fueled by natural gas on an already existing compressor station site surrounded by primarily agricultural grazing land. The project scope would not require a large surface area of disturbance that would create a concern for storm water. Contracted construction will be responsible for installing and maintaining erosion and sediment transport control and use best management practices during the construction phase. Company staff would oversee maintenance and operations of equipment on site, as needed. |
| Solid Waste | Some solid waste could be generated during construction activities, but once operational any solid waste generation would be expected to be minimal |
| Cultural Resources | The property is already in use as agricultural property and no effects on cultural resources would be expected because of the proposed action. |
| Hazardous Substances | The applicant proposes installing and operating a natural gas pipeline compressor station fueled by natural gas and not likely to contribute to the release of any hazardous substances. The applicant must comply with all applicable local, county, state and federal requirements pertaining to hazardous substances. |
| Reclamation | The applicant proposes to install and operate a natural gas pipeline compressor station fueled by natural gas on an area of approximately 4 acres (~450 sq ft) |

Table 3. Cumulative Impacts

| Past Actions | This is a new pre-construction permit; however, another compressor pipeline facility known as Blaine County #3 (BC3) is co-located on site. The compressor pipeline station is owned by the Havre Pipeline Company (HPC), a subsidiary of NWE. |
| --- | --- |
| Present Actions |  | The applicant proposes installing and operating two (2) natural gas-fired 1900 bhp engines to boost the pressure of a natural gas pipeline. |
| Related Future Actions |  | No future actions are foreseen at this site. As application, any future actions would require environmental review pursuant to MEPA. |

### Purpose, Need, and Benefits

DEQ's purpose in conducting this environmental review is to act upon NWE’s application for an MAQP to boost natural gas pipeline pressures. DEQ’s action on the permit application is governed by § 75-2-201, et seq., Montana Code Annotated (MCA) and the Administrative Rules of Montana (ARM) 17.8.740, et seq.

The applicant’s purpose and need, as expressed to DEQ in seeking this action, is to construct and operate a natural gas pipeline compressor station utilizing gas from the pipeline to power two (2) 1,900 bhp natural gas-fired engines.

Figure 1. General Location of the Proposed Project

Inset middle left shows an expanded image of the facility and the blue shape is the proposed building location.

### Other Governmental Agencies and Programs with Jurisdiction

The proposed action would be located on primarily agricultural grazing with dispersed oil and gas facilities. All applicable local, state, and federal rules must be adhered to, which may include other local, state, federal, or tribal agency jurisdiction. Other governmental agencies which may have overlapped, or additional jurisdiction include but may not be limited to: Montana Board of Oil and Gas.

## Evaluation of Affected Environment And Impact by Resource

The impact analysis will identify and evaluate the proximate direct and secondary impacts TO THE PHYSICAL ENVIRONMENT AND POPULATION IN THE AREA TO BE AFFECTED BY THE PROPOSED PROJECT. Direct impacts occur at the same time and place as the action that causes the impact. Secondary impacts are a further impact to Montana’s environment that may be stimulated, induced by, or otherwise result from a direct impact of the action (ARM 17.4.603(18)). Where impacts would occur, the impacts will be described in this analysis. When the analysis discloses environmental impacts, these are proximate impacts pursuant to 75-1-201(1)(b)(iv)(A), MCA.

Cumulative impacts are the collective impacts on Montana’s environment within the borders of Montana of the Proposed Action when considered in conjunction with other past and present actions related to the Proposed Action by location and generic type. Related future actions must also be considered when these actions are under concurrent consideration by any state agency through pre-impact statement studies, separate impact statement evaluation, or permit processing procedures (ARM 17.4.603(7)). The project identified in Table 1 was analyzed as part of the cumulative impacts assessment for each resource subject to review, pursuant to MEPA (75-1-101, et. seq).

The duration of the proposed action is quantified as follows:

- Construction Impacts (short-term): These are impacts to the environment that would occur during the construction period, including the specific range of time.

- Operation Impacts (long-term): These are impacts to the environment during the operational period of the proposed action, including the anticipated range of operational time.

The intensity of the impacts is measured using the following:

- No impact: There would be no change from current conditions.

- Negligible: An adverse or beneficial effect would occur but would be at the lowest levels of detection.

- Minor: The effect would be noticeable but would be relatively small and would not affect the function or integrity of the resource.

- Moderate: The effect would be easily identifiable and would change the function or integrity of the resource.

- Major: The effect would alter the resource.

### Geology and Soil Quality, Stability and Moisture

This section includes the following resource areas, as required in ARM 17.4.609: Geology; Soil Quality, Stability, and Moisture

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the Havre Pipeline Company (HPC) Blaine County #3 Compressor Station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 brake horsepower (bhp) and a 261 bhp natural gas-fueled emergency generator. According to the USDA Soil Survey the affected soil environment is over 60% of the following: Perma-Castner-Belain complex, Castern-Perma-rock outcrop, Hedoes-Belain-Castner and Hedoes-Belain loams ranging from 15-70 percent slopes. The remaining soil environment is a mixture of over 30 different types of gravely and clay loams with a slope of 0-8 percent. The overall slope of the land in the area is approximately 10%. The majority of soil types in this location are well-drained.

Direct Impacts

Proposed Action: The construction for the new facility footprint would cover approximately 1.3 acres of land which is already in use by HPC for the Blaine County #3 compressor station. Heavy construction equipment would temporarily be onsite and may disturb land during staging and construction. Direct impacts to geology, soil quality, stability and moisture content would likely be minor and negligible.

Secondary Impacts

Proposed Action: After construction the land may be occasionally disturbed by vehicles and heavy equipment performing maintenance. Secondary impacts to geology, soil quality, stability and moisture content would likely be minor and negligible.

Cumulative Impacts

Proposed Action: Cumulative impacts would result in disturbed land of approximately 1 acre of leveled land on a parcel with an approximate 10% slope. The cumulative impacts to geology, soil quality, stability and moisture content would likely be minor and negligible.

### Water Quality, Quantity, And Distribution

This section includes the following resource areas, as required in ARM 17.4.609: Water Quality, Quantity and Distribution

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. The proposed project appears to have an unnamed ephemeral creek approximately 500 feet to the northeast of the proposed building location based on google satellite imaging. The nearest named creeks are Hedge Creek located approximately 1.3 miles north and Cow creek located approximately 1.0 miles west of the proposed construction site. Total annual precipitation for the area is approximately 14.8 inches on average.

Direct Impacts

Proposed Action: Construction of the facility would not result in discharges of ground or surface water. The applicant has acquired and would be required to implement a Storm Water Pollution Prevention Plan (SWPPP) from DEQ. Direct impacts to the water quality, quantity and distribution for the immediate area would be minor during construction.

Secondary Impacts

Proposed Action: Following construction, the expected footprint of this facility would be approximately 1.3 acres of graveled surface. Precipitation runoff of the immediate area may be slowed by the reduced ground slope. Use of the facility would be expected to generate minimal amounts of fugitive dust, which would be controlled, as necessary, by the use of water. The water would be sourced off site and transported to the facility. Secondary impacts to the water quality, quantity and distribution for the immediate area would be minor.

Cumulative Impacts

Proposed Action: The proposed project would occur on site with an adjacent compressor station with adjacent agricultural and grazing lands. Existing oil and gas operations require reasonable precautions for the control of fugitive dust from unpaved roads. Cumulative impacts to water quality, quantity and distribution may be negligible to minor.

### Air Quality

This section includes the following resource areas, as required in ARM 17.4.609: Air Quality

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC facility. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. The air quality of the area around the proposed project is unclassifiable or in compliance with the applicable National Ambient Air Quality Standards (NAAQS). No significant point sources of air pollution located in the area are affected by the proposed project. Existing sources of air pollution in the area are limited and generally include oil and gas facilities similar to the proposed project, fugitive dust associated with high wind events and exposed ground, vehicle traffic on paved and unpaved roads (fugitive dust), vehicle exhaust emissions, and various agricultural practices (vehicle exhaust emissions and fugitive dust).

The nearest Class I airshed is the UL Bend Wilderness located southeast of the proposed project in the Upper Missouri Breaks are located approximately 65 miles to the southwest of the proposed facility.

Applicants are required to comply with all laws related to air, such as the Federal Clean Air Act, NAAQS as set by the Environmental Protection Agency (EPA), and the Clean Air Act of Montana.

In addition, MAQP #5358-00 provides legally enforceable conditions regarding the emitting units themselves, pollution controls, and requires the applicant to take reasonable precautions to limit fugitive dust from this location.

Direct Impacts

Proposed Action: Fugitive dust emissions resulting from the construction of this proposed facility may adversely impact air quality. However, NWE must use reasonable precautions to limit fugitive dust generation during construction activities. Further, no air quality restrictions exist for the affected area. Therefore, the proposed project would not be expected to cause or contribute to a NAAQS violation for particulate matter (fugitive dust) generated during construction. Any direct impacts would be short-term, negligible, consistent with existing impacts and mitigated by implementation of enforceable, reasonable precautions for fugitive dust created by construction and associated with vehicle traffic on unpaved roads.

Secondary Impacts

Proposed Action: The proposed project is to operate two (2) Waukesha L7044GSI natural gas-fired compressor engines up to 1900 bhp for up to 8,760 hours per year (continuous operation) and a 261 bhp natural gas-fired emergency generator to operate a compressor station boosting natural gas pipeline pressures. The emission inventory, located in Section IV of the MAQP analysis, is based on emission factors provided by the manufacturer of the engines, EPA’s AP-42 emission factors guidance, and on limits deemed Best Available Control Technology (BACT), as required. Controlled emissions would not be expected to cause or contribute to a violation of the National Ambient Air Quality Standards (NAAQS).

Adverse air quality impacts would be minor because of the proposed project. See permit analysis for more information regarding air quality impacts. The majority of pollutants from the proposed project would be related to the combustion of natural gas. This would result in the release of NOx, CO, SOx, VOC’s and particulate matter.

Emissions from the proposed project require the use of BACT and would not be expected to contribute to a violation of health and welfare-based primary and secondary NAAQS. Primary NAAQS provide public health protection, including protecting the health of “sensitive” populations such as asthmatics, children, and the elderly. Secondary NAAQS provide public welfare protection, including protection against decreased visibility, damage to animals, crops, vegetation and building. See permit analysis for more detailed information regarding air quality impacts. Any adverse impacts would be long-term and minor.

Cumulative Impacts

Proposed Action: Cumulative impacts from the operation of the facility include emissions from the Blaine County #3 Compressor Station as well as the proposed emissions from the new facility. A single source determination, as discussed in the permit analysis, has identified that both the NWE Bear Paw compressor station and HPC are considered a single source of emissions. Collectively, the potential to emit from this facility is below Title V thresholds, and the permit holders are still required to pay the required fees as separate entities. See permit analysis and emission inventory within the MAQP #5358-00 for additional information.

Cumulative impacts from the operations of the facility are restricted by the permit conditions and limits within the MAQPs; therefore, any expected air quality impacts would be minor. Blaine County also has other station sources, many are similar pipeline compressors for oil and gas transmission, all contribute to the overall air quality in Blaine County. Any cumulative impacts from the proposed source would be long-term and minor.

### Vegetation Cover, Quantity, and Quality

This section includes the following resource areas, as required in ARM 17.4.609: Vegetation Cover, Quantity and Quality

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC Compressor Station. The new facility would disturb approximately 1.3 acres of ground for staging, construction and operation. A report was generated from the Montana Natural Heritage Program (MTNHP) with a file download performed on June 17, 2026. The polygon selected was the immediate area surrounding the proposed site.

Vegetation in the affected area is primarily lowland and prairie grassland with conifer-dominated forest and woodlands. According to the MTNHP data search for vascular plants, there are eleven species of concern (SOC) for the immediate surrounding area. See table below:

| Common Name | Scientific Name | MT Status | Habitat |
| --- | --- | --- | --- |
| Schweinitz's Flatsedge | Cyperus schweinitzii | SOC | Sandy sites |
| Silver Bladderpod | Physaria ludoviciana | SOC | Sandy sites |
| Long-sheath Waterweed | Elodea bifoliata | SOC | Wetland/Riparian (Shallow water) |
| Double Bladderpod | Physaria brassicoides | SOC | Breaklands/badlands |
| Heart-leaved Buttercup | Ranunculus cardiophyllus | SOC | Grasslands (Moist, Montane) |
| Slim-pod Venus'-looking-glass | Triodanis leptocarpa | SOC | Grasslands |
| Smooth Goosefoot | Chenopodium subglabrum | SOC | Sandy sites |
| Wood Lily | Lilium philadelphicum | SOC | Moist, usually calcareous, soils in meadows, grasslands, fens, and woodlands |
| Crawe's Sedge | Carex crawei | SOC | Wetland/Riparian |
| Scribner's Ragwort | Senecio integerrimus var. scribneri | SOC | Sagebrush shrublands and grasslands, usually with clay soils |
| Western Moonwort | Botrychium hesperium | SOC | Various Mesic Sites |

Direct Impacts

Proposed Action: Construction of the proposed facility would take place on pre-existing disturbed oil and gas infrastructure land and nearby agricultural and grazing lands. The proposed groundwork and installation would require approximately 1.3 acres of land disturbance with the co-located HPC gas and oil facility. The land disturbance would extend the current fence line to encompass the proposed facility. Direct impacts to vegetation beyond agricultural and grazing lands would likely be negligible and minor.

Secondary Impacts

Proposed Action: The land use directly adjacent to the facility is currently used for agricultural and grazing purposes. Secondary impacts to vegetation would likely be negligible and minor. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings

Cumulative Impacts

Proposed Action: The land use directly adjacent to the facility is currently used for agricultural and grazing purposes. Cumulative impacts to vegetation would likely be negligible and minor. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings

### Terrestrial, Avian, and Aquatic Life and Habitats

This section includes the following resource areas, as required in ARM 17.4.609: Terrestrial and Aquatic Life and Habitats; Unique, Endangered, Fragile, or Limited Environmental Resources

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located with an HPC gas and oil facility. The new facility would disturb approximately 1.3 acres of ground for staging, construction and operation, including an extension of the current fence line extending the facility into land currently used for agriculture and grazing purposes.

| Common Name | Scientific Name | MT Status | Habitat |
| --- | --- | --- | --- |
| Greater Sage-Grouse | Centrocercus urophasianus | SOC | Sagebrush |
| Golden Eagle | Aquila chrysaetos | SOC | Grasslands |
| Clark's Nutcracker | Nucifraga columbiana | SOC | Conifer forest |
| Ovenbird | Seiurus aurocapilla | PSOC | Deciduous forest |
| Veery | Catharus fuscescens | SOC | Riparian forest |
| Sprague's Pipit | Anthus spragueii | SOC | Grasslands |
| Sharp-tailed Grouse | Tympanuchus phasianellus | SOC | Shrub grassland |
| Common Poorwill | Phalaenoptilus nuttallii | PSOC | Shrub grassland |
| Ferruginous Hawk | Buteo regalis | SOC | Sagebrush grassland |
| Horned Grebe | Podiceps auritus | SOC | Wetlands |
| Long-billed Curlew | Numenius americanus | SOC | Grasslands |
| Red-headed Woodpecker | Melanerpes erythrocephalus | SOC | Riparian forest |
| Cassin's Finch | Haemorhous cassinii | PSOC | Drier conifer forest |
| American Bittern | Botaurus lentiginosus | SOC | Wetlands |
| American White Pelican | Pelecanus erythrorhynchos | SOC | Lakes, ponds, reservoirs |
| Brewer's Sparrow | Spizella breweri | SOC | Sagebrush |
| Bobolink | Dolichonyx oryzivorus | SOC | Moist grasslands |
| Great Blue Heron | Ardea herodias | SOC | Riparian forest |
| Thick-billed Longspur | Rhynchophanes mccownii | SOC | Grasslands |
| American Goshawk | Astur atricapillus | SOC | Mixed conifer forests |
| Mountain Plover | Anarhynchus montanus | SOC | Grasslands |
| Bald Eagle | Haliaeetus leucocephalus | SSS | Riparian forest |
| Baird's Sparrow | Centronyx bairdii | SOC | Grasslands |
| Indiscriminate Cuckoo Bumble Bee | Bombus insularis | PSOC | Alpine, montane meadows, shrub steppe, prairie grassland |
| Western Bumble Bee | Bombus occidentalis | SOC | Urban, montane/steppe grassland and shrubland |
| Suckley's Cuckoo Bumble Bee | Bombus suckleyi | SOC | Montane/steppe grassland and shrubland |
| Monarch | Danaus plexippus | SOC | Milkweed Milkweed |
| American Bumble Bee | Bombus pensylvanicus | SOC | Fields, farmland, grasslands |
| Northern Leopard Frog | Lithobates pipiens | SOC | Wetlands, floodplain pools |
| Preble's Shrew | Sorex preblei | SOC | Sagebrush grassland |
| Dwarf Shrew | Sorex nanus | PSOC | Rocky habitat |
| Merriam's Shrew | Sorex merriami | SOC | Sagebrush grassland |
| Long-eared Myotis | Myotis evotis | SOC | Forest |
| Long-legged Myotis | Myotis volans | SOC | Conifer forest |
| Silver-haired Bat | Lasionycteris noctivagans | SOC | Riparian and forest |
| Eastern Red Bat | Lasiurus borealis | SOC | Riparian forest |
| Northern Hoary Bat | Lasiurus cinereus | SOC | Riparian and forest |
| Townsend's Big-eared Bat | Corynorhinus townsendii | SOC | Caves in forested habitats |
| Spotted Bat | Euderma maculatum | PSOC | Cliffs with rock crevices |
| Little Brown Myotis | Myotis lucifugus | SOC | Generalist |
| North American Porcupine | Erethizon dorsatum | PSOC | Mixed forest |
| Prairie Shrew | Sorex haydeni | PSOC | Grasslands |
| Black-tailed Prairie Dog | Cynomys ludovicianus | SOC | Grasslands |
| Greater Short-horned Lizard | Phrynosoma hernandesi | SOC | Sandy / gravelly soils |

- SOC- Species of Concern

- PSOC – Potential Species of Concern

- SSS – Special Species Status

Direct Impacts

Proposed Action: Construction of the proposed facility would take place on pre-existing disturbed land and adjacent agricultural and grazing lands. The proposed groundwork and installation would require approximately 1.3 acres of land disturbance with the co-located with an existing HPC compressor station. The land disturbance would extend the current fence line to encompass the proposed facility. Any habitat directly adjacent to this facility appears to be used for agricultural and grazing land purposes. Direct impacts to these habitats are likely negligible and minor.

Secondary Impacts

Proposed Action: Secondary impacts to the terrestrial, avian and aquatic life habitats would be minor to negligible as the new construction would be co-located on an already existing oil and gas facility and adjacent agricultural and grazing lands. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings

Cumulative Impacts

Proposed Action: Cumulative impacts to the terrestrial, avian and aquatic life habitats would be minor to negligible as the new construction would be co-located on an already existing oil and gas facility and adjacent agricultural and grazing lands. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings

### Unique, Endangered, Fragile, or Limited Environmental Resources

This section includes the following resource areas, as required in ARM 17.4.609: Unique, Endangered, Fragile, or Limited Environmental Resources.

Affected Environment

The proposed construction and operation of Bear Paw Compressor station is on the same site as the HPC Blaine County #3 compressor station. As described in Section(s) 4 and 5 above, DEQ conducted a search using the NTNHP website. The query used a polygon search that included the site location and surrounding polygons.

The MTNHP search did identify the Greater Sage Grouse as a species of concern. Furthermore, this area is located in general sage grouse habitat as defined by executive order no 12-2025. As the application for this project was received after the executive order effective date of January 1, 2016, the proposed project is subject to review under the Executive Order. AS required under the executive order, the proposed project was reviewed by the Montana Sage Grouse Oversight Team (MSGOT) and that information was submitted by the applicant to DEQ.

NWE opted to make a contribution to the Stewardship account, as allowed by the Stewardship Act, instead of developing a permittee-responsible package to offset potential impacts. A total payment of $518.86 was assessed for the permitted action. Funds were deposited by the NWE on May 22, 2026, to the Stewardship account. MSGOT awards these funds through the Stewardship Account grant process to conserve habitat and sage grouse population in North Central Montana Sage Grouse Service Area Project.

Direct Impacts

Proposed Action: Any potential impacts to sage grouse habitat would be long term and minor to moderate, because the proposed action would require new disturbance during construction of the proposed project. However, because the proposed action would occur within an existing industrial site with adjacent agricultural and grazing lands, it is unlikely sage grouse would occupy or use the site for all or part of their life cycle.

Secondary Impacts

Proposed Action: Any secondary impacts to sage grouse habitat would be long term and minor based on the proximity to the nearest lek, which is located over four miles to the away to the southeast. Further, because the proposed action would be consistent with existing industrial operations within the affected area, it is unlikely sage grouse would occupy the affected habitat.

Cumulative Impacts

Proposed Action: Any cumulative impacts to sage grouse habitat would be long term and negligible because the affected area has already been disturbed by industrial activities. Therefore, any cumulative impacts would be consistent with existing impacts. Further, because the proposed action would be consistent with existing industrial operations within the affected area, it is unlikely sage grouse would occupy the affected habitat.

### Historical and Archaeological Sites

This section includes the following resource areas, as required in ARM 17.4.609: Historical and Archaeological Sites

Affected Environment

The proposed construction and operation of Bear Paw Compressor station is on the same site as the HPC Compressor Station. A State Historic Preservation Office (SHPO) report was requested on May 20, 2026, for the site location. According to SHPO, “there would be no cultural or historic properties affected by this undertaking.” Approximately 2000 ft to the northeast of the proposed facility on google maps, there is a building labeled as the “Cow Trail Island School (Historic)” According to www.publicschoolreview.com the school closed in 2004. A follow-up inquiry with SHPO dated June 23rd, 2026, noted that the schoolhouse is located on “private land” and that the construction “is not within the area of potential effect”

Direct Impacts

Proposed Action: There are no anticipated direct impacts to historical and or archaeological sites according to SHPO that would result from the construction of this facility.

Secondary Impacts

Proposed Action: There are no anticipated secondary impacts to historical and/or archaeological sites according to SHPO that would result from the construction of this facility. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings

Cumulative Impacts

Proposed Action: There are no anticipated cumulative impacts to historical and/or archaeological sites according to SHPO that would result from the construction of this facility. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings

### Aesthetics

This section includes the following resource areas, as required in ARM 17.4.609: Aesthetics

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. This site is surrounded by primarily agricultural and grazing land with dispersed oil and gas facilities nearby.

Direct Impacts

Proposed Action: The proposed project would impact on the visual and auditory aesthetics in the affected area. Construction would require approximately 1.3 acres of associated groundwork. Construction is expected to last approximately 3 months. This disturbance is co-located with another compressor station. Because the proposed project is consistent with existing and regulated oil and gas operations in the affected area, any direct impacts from the construction of the proposed facility would be short-and long-term, consistent with existing impacts and negligible.

Secondary Impacts

Proposed Action: Operation of the facility would potentially impact nearby auditory aesthetics in the affected area. The sound from operation would be minimized via structural enclosures and a catalytic silencer which reduces noise from the exhaust across frequencies spanning 31.5 to 8000 Hz. Because the proposed project is consistent with existing and regulated oil and gas operations in the affected area, any secondary impacts from the operation of the proposed facility would be short-and long-term, consistent with existing impacts and negligible.

Cumulative Impacts

Proposed Action: Operation of the facility include both the Bear Paw Compressor and the Havre Pipeline Compressor – both are co-located on the same site, and both would contribute to the visible and auditory aesthetics. Because the proposed project is consistent with existing and regulated oil and gas operations in the affected area, any cumulative impacts from the construction of the proposed facility would be short-and long-term, consistent with existing impacts and negligible.

### Demands on Environmental Resources of Land, Water, Air, or Energy

This section includes the following resource areas, as required in ARM 17.4.609: Demands on Environmental Resources of Land, Water, Air, or Energy

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC Compressor Station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. The facility would be powered from the natural gas pipeline it is servicing.

Direct Impacts

Proposed Action: Some direct impacts to land, water and air would be expected because of the proposed project identified by the corresponding impacts analysis above. Construction of the proposed facility would involve limited operation of heavy equipment, and the combustion of fossil fuels would be required for the operation of such equipment. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings. Any adverse direct impacts related to construction would be short-term and negligible. No beneficial direct impacts would be expected because of the proposed project.

Secondary Impacts

Proposed Action: Some secondary impacts to land, water and air would be expected because of the proposed project as identified by the corresponding impacts analysis as highlighted in previous sections. Further, the proposed facility would be powered by natural gas from the pipeline that the facility is compressing. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings. Any adverse secondary impacts related to environmental demands related to operation would be short and long-term negligible to minor.

Cumulative Impacts

Proposed Action: Cumulative impacts to land, water and air would be expected because of the proposed project as identified by the corresponding impacts analysis as highlighted in previous sections. Compressor stations use the fuel they are compressing to improve transmission of that fuel to end users. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings. Any adverse secondary impacts related to environmental demands related to operation would be short and long-term negligible to minor.

### Impacts on Other Environmental Resources

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Other Environmental Resources

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC Compressor Station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. The proposed project is small by industrial standards and is located in an area with a co-existing compressor station and with dispersed nearby oil and gas facilities and agricultural lands.

Direct Impacts

Proposed Action: Fugitive dust emissions resulting from the construction of the proposed facility may adversely affect air quality in the area. However, NWE must use reasonable precautions to limit fugitive dust generated from construction activities; therefore, the proposed project would not be expected to contribute to a violation of the applicable National Ambient Air Quality Standards (NAAQS) for particulate matter. See permit analysis for more detailed information regarding air quality impacts. Secondary NAAQS provide public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings. Therefore, any adverse direct impact to other environmental resources would be short-term and minor. No beneficial direct impacts would be expected because of the proposed project.

Secondary Impacts

Proposed Action: Maintenance of the facility will be periodically undertaken, which might include the need for heavy equipment and vehicles. These vehicles may result in temporary and periodic increases of fugitive dust, and vehicular emissions. However, the maintenance is not expected to contribute to a violation of the applicable NAAQS. Secondary NAAQS provides public welfare protection, including protection against decreased visibility and damage to animals, wildlife, crops, vegetation and buildings. Therefore, any adverse secondary impacts would to other environmental resources would be periodic, short-term and minor.

Cumulative Impacts

Proposed Action: No other environmental resources, beyond the anticipated resource areas already covered within the EA would result in any known additional cumulative impacts.

### Human Health and Safety

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Human Health and Safety

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. The applicant would be required to adhere to all applicable state and federal safety laws. The Occupational Safety and Health Administration (OSHA) has developed rules and guidelines to reduce the risks associated with this type of labor. Few, if any, members of the public would be in immediate proximity to the project during construction or operations.

Direct Impacts

Proposed Action: Construction would be required to adhere to all applicable state and federal safety laws. OSHA has developed rules and guidelines to reduce the risks associated with this type of labor. Few, if there were any members of the public would be in immediate proximity to this project during construction. Any direct impacts would be minor and negligible.

Secondary Impacts

Proposed Action: Operation of the facility would be required to adhere to all applicable state and federal safety laws. OSHA has developed rules and guidelines to reduce the risks associated with this type of labor. Few, if there were any members of the public would be in immediate proximity to this project during operation. Any secondary impacts would be minor and negligible.

Cumulative Impacts

Proposed Action: Operation of the facility would be required to adhere to all applicable state and federal safety laws. OSHA has developed rules and guidelines to reduce the risks associated with this type of labor. Few, if there were any members of the public would be in immediate proximity to this project during operation. Any cumulative impacts would be minor and negligible.

### Industrial, Commercial, and Agricultural Activities and Production

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Industrial, Commercial, and Agricultural Activities and Production

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator. The proposed project is small by industrial standards and is located in an area with a co-existing compressor station and with dispersed nearby oil and gas facilities and agricultural lands.

Direct Impacts

Proposed Action: Construction would displace approximately 1.3 acres of grazing land directly to the north of the current HPC Blaine County Compressor Station and extend the overall fence line of the facility during construction. Any direct impacts regarding agricultural activities would be minor and negligible. Direct impacts for industrial natural gas pipeline transmission would be minor and beneficial.

Secondary Impacts

Proposed Action: Operation of the facility to would be enclosed in a fence to separate the facility from the agricultural and grazing lands. Approximately 1.3 acres of land would no longer be available for agricultural and grazing use. Any adverse secondary impacts regarding agricultural activities would be minor and negligible. Secondary impacts for industrial natural gas pipeline transmission would be minor and beneficial.

Cumulative Impacts

Proposed Action: Cumulative impacts to agricultural production would result in a negligible loss of agricultural and grazing land. Cumulative impacts for gas and oil would be negligible and beneficial in terms of natural gas transmission efficiency.

### Quantity and Distribution of Employment

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Quantity and Distribution of Employment

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

The applicant identified that construction and operation of this facility would not be an expected change in current employment for the company

Direct Impacts

Proposed Action: Negligible, short-term and beneficial impacts from installation may be expected through the employment of contractors to install the facility.

Secondary Impacts

Proposed Action: Negligible secondary impacts expected to the construction or operation of the facility are expected. No secondary employment is expected as a result of the proposed action.

Cumulative Impacts

Proposed Action: Negligible cumulative impacts expected to the construction or operation of the facility are expected. No secondary employment is expected as a result of the proposed action.

### Local and State Tax Base and Tax Revenue

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Local and State Tax Base and Tax Revenue

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: No direct impacts would be expected because of the proposed action. Installation may increase the local sales of goods and services during construction. However, because the proposed project would be approximately 3 months, the direct impacts to the local and state tax base would be long-term and minor.

Secondary Impacts

Proposed Action:  No secondary impacts would be expected because of the proposed project. Local, state and federal governments would be responsible for appraising the property, setting tax rates, collecting taxes from the company, employees, or landowners benefiting from the proposed action. Further NWE would be responsible for accommodation of any increased taxes associated with operating the proposed facility. Therefore, any secondary impacts would be negligible to minor, consistent with existing impacts in the area.

Cumulative Impacts

Proposed Action: Long-term negligible, beneficial to minor impacts to local and state tax base and tax revenues are anticipated from this permitting action.

### Demand for Government Services

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Demands for Government Services

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: No direct impacts would be expected because of the proposed project. The air quality permit has been prepared by state government employees as part of their day-to-day, regular responsibilities. Therefore, any adverse direct impacts to demands for government services is consistent with existing impacts and negligible.

Secondary Impacts

Proposed Action: No beneficial secondary impacts would be expected because of the proposed project. Following construction of the proposed facility, initial and ongoing compliance inspections of facility operations would be accomplished by state government employees as part of their typical, regular duties and required to ensure the facility is operating within the limits and conditions listed in the air quality permit. Therefore, any adverse secondary impacts to demands for government services would be negligible and consistent with existing impacts in the affected area.

Cumulative Impacts

Proposed Action: Minor cumulative impacts are anticipated on government services with the proposed action and a minimal increase in impact would occur, but regulators would likely combine visits to cover regulatory oversight needs.

### Locally Adopted Environmental Plans and Goals

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Locally Adopted Environmental Plans and Goals

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

DEQ has revied the Blaine County website and found no locally adopted environmental plans and goals for the area.

Direct Impacts

Proposed Action: No direct impacts are expected because of the project. No locally adopted environmental plans or goals were identified.

Secondary Impacts

Proposed Action: No secondary impacts are expected because of the project. No locally adopted environmental plans or goals were identified.

Cumulative Impacts

Proposed Action: No cumulative impacts are expected because of the project. No locally adopted environmental plans or goals were identified.

### Access to and Quality of Recreational and Wilderness Activities

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Access to and Quality of Recreation and Wilderness Activities

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: No direct construction or operational impacts to access to and quality of recreational wilderness activities because the facility is located on private property with no public access to the immediate surrounding area.

Secondary Impacts

Proposed Action: No secondary construction or operational impacts to access to and quality of recreational wilderness activities because the facility is located on private property with no public access to the immediate surrounding area.

Cumulative Impacts

Proposed Action: No cumulative construction or operational impacts to access to and quality of recreational wilderness activities because the facility is located on private property with no public access to the immediate surrounding area.

### Density and Distribution of Population and Housing

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Density and Distribution of Population and Housing

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: Negligible impact on the overall distribution of employment would be expected as there is no increase in employment.

Secondary Impacts

Proposed Action: Negligible impact on the overall distribution of employment would be expected as there is no increase in employment.

Cumulative Impacts

Proposed Action: Negligible impact on the overall distribution of employment would be expected as there is no increase in employment.

### Social Structures and Mores

This section includes the following resource areas, as required in ARM 17.4.609: Impacts on Social Structures and Mores

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the Havre Pipeline Company’s Blaine County #3 Compressor Station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: The proposed action is location on a previously existing industrial site and no changes or disruptions to native or traditional lifestyles are expected because of this project. Therefore there are no direct impacts to social structures and mores.

Secondary Impacts

Proposed Action: The proposed action is location on a previously existing industrial site and no changes or disruptions to native or traditional lifestyles are expected because of this project. Therefore there are no secondary impacts to social structures and mores.

Cumulative Impacts

Proposed Action: The proposed action is location on a previously existing industrial site and no changes or disruptions to native or traditional lifestyles are expected because of this project. Therefore there are no cumulative impacts to social structures and mores.

### Cultural Uniqueness and Diversity

This section includes the following resource areas, as required in ARM 17.4.609: Impacts to Cultural Uniqueness and Diversity

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: No direct impacts to the existing cultural uniqueness and diversity of the affected population would be expected because of the proposed project. The proposed project would be constructed in a relatively short period of time, no direct impacts to cultural uniqueness and diversity are expected.

Secondary Impacts

Proposed Action No direct impacts to the existing cultural uniqueness and diversity of the affected population would be expected because of the proposed project. The proposed project would not be expected to result in an increase or decrease in the local population as this facility would most likely utilize the employees who already work on site.

Cumulative Impacts

Proposed Action: No direct impacts to the existing cultural uniqueness and diversity of the affected population would be expected because of the proposed project. The proposed project would not be expected to result in an increase or decrease in the local population as this facility would most likely utilize the employees who already work on site.

### Private Property Impacts

The proposed project would take place on private land owned by the applicant. DEQ’s approval of NWE’s MAQP affect the applicant’s real property. DEQ has determined, however, that the permit conditions are reasonably necessary to ensure compliance with applicable requirements. Therefore, DEQ’s approval of NWE’s MAQP would not have private property-taking or damaging implications.

### Other Appropriate Social and Economic Circumstances

This section includes the following resource areas, as required in ARM 17.4.609: Impacts to Other Appropriate Social and Economic Circumstances

Affected Environment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

Direct Impacts

Proposed Action: DEQ is unaware of any other appropriate short-term social and economic circumstances in the affected area that may be directly impacted by the proposed project. Due to the nature of the proposed action, no further direct impact would be expected because of the proposed project.

Secondary Impacts

Proposed Action: DEQ is unaware of any other appropriate long-term social and economic circumstances in the affected area that may be impacted by the proposed project. No further secondary impacts would be expected because of the proposed project.

Cumulative Impacts

Proposed Action: No cumulative impacts to other appropriate social and economic circumstances are anticipated since no direct impacts or secondary impacts were identified.

### Greenhouse Gas Assessment

NWE proposes to construct and operate a new natural gas compressor station referred to as the Bear Paw Compressor Station co-located on site with the HPC compressor station. The new facility would install two Waukesha L7044GSI compressor engines up to 1,900 bhp and a 261 bhp natural gas-fueled emergency generator.

The analysis area for this resource is limited to the activities regulated by the issuance of NWE’s MAQP #5358-00 permit which is construction and operation of the Bear Paw Compressor Station that is co-located and considered a single-source with HPC’s Blaine County #3 Compressor Station as determined by same owner (NWE), same SIC-code (4922-98) and co-located (contiguous and adjacent). The amount of natural-gas fuel utilized at this site may be impacted by a number of factors including seasonal weather impediments and equipment malfunctions. To account for these factors DEQ has calculated the range of emissions using the applicant’s potential to emit.

For the purpose of this analysis, DEQ has defined greenhouse gas emissions as the following gas species: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), and many species of fluorinated compounds. The range of fluorinated compounds includes numerous chemicals which are used in many household and industrial products. Other pollutants can have some properties that also are similar to those mentioned above, but the EPA has clearly identified the species above as the primary GHGs.  Water vapor is also technically a greenhouse gas, but its properties are controlled by the temperature and pressure within the atmosphere, and it is not considered an anthropogenic species.

DEQ has calculated GHG emissions using the EPA Simplified GHG Calculator version May 2023, for the purpose of totaling GHG emissions. This tool totals carbon dioxide (CO2), nitrous oxide (N2O), and methane (CH4) and reports the total as CO2 equivalent (CO2e) in metric tons CO2e. The calculations in this tool are widely accepted to represent reliable calculation approaches for developing a GHG inventory. DEQ has determined EPA’s Scope 1 GHG impacts as defined in the Inventory Guidance for Greenhouse Gas Emissions are appropriate under MEPA for this Proposed Action. Scope 1 emissions are defined as direct GHG emissions that occur from sources that are controlled or owned by the organization (EPA Center for Corporate Climate Leadership). DEQ’s review of Scope 1 emissions is consistent with the agency not evaluating downstream effects of other types of impacts.

This review does not include an assessment of GHG impacts in quantitative economic terms, otherwise known as evaluating the social cost of carbon. DEQ instead calculates potential GHG emissions and provides a narrative description of GHG impacts. This approach is consistent with Montana Supreme Court caselaw and the agency’s discussion of other impacts in this EA. See Belk v. Mont. DEQ, 2022 MT 38, ¶ 29.

Secondary Impacts

GHG emissions contribute to changes in atmospheric radiative forcing, resulting in climate change impacts. GHGs act to contain solar energy loss by trapping longer wave radiation emitted from the Earth’s surface and act as a positive radiative forcing component (BLM 2021).

Per EPA’s website “Climate Change Indicators”, the lifetime of carbon dioxide cannot be represented with a single value because the gas is not destroyed over time. The gas instead moves between air, ocean, and land mediums with atmospheric carbon dioxide remaining in the atmosphere for thousands of years, due in part to the very slow process by which carbon is transferred to ocean sediments. Methane remains in the atmosphere for approximately 12 years. Nitrous oxide has the potential to remain in the atmosphere for about 109 years (EPA, Climate Change Indictors). The impacts of climate change throughout the north central region of Montana include changes in flooding and drought, rising temperatures, and the spread of invasive species (BLM 2021).

Applicant estimates that up to 265,280 MMBtu of Natural Gas would be used to power the Bear Paw Compressor Station based on potential to emit and emission inventory calculations. According to the EPA GHG tool, the yearly operation of the Bear Paw Compressor Station would account for approximately 14,090 metric tons of CO2e in the atmosphere. Montana accounts for 51.04 million metric tons of CO2e based on the EPA SIT for the year 2022. This action constitutes approximately 0.0276% of the total CO2e annual emissions

Cumulative Impacts

Montana recently used the EPA State Inventory Tool (SIT) to develop a greenhouse gas inventory in conjunction with preparation of a possible grant application for the Community Planning Reduction Grant (CPRG) program. This tool was developed by EPA to help states develop their own greenhouse gas inventories, and this relies upon data already collected by the federal government through various agencies. The inventory specifically deals with carbon dioxide, methane, and nitrous oxide and reports the total as CO2e. The SIT consists of eleven Excel based modules with pre-populated data that can be used with default settings or in some cases, allows states to input their own data when the state believes their own data provides a higher level of quality and accuracy. Once each of the eleven modules is filled out, the data from each module is exported into a final “synthesis” module which summarizes all of the data into a single file. Within the synthesis file, several worksheets display the output data in a number of formats such as GHG emissions by sector and GHG emissions by type of greenhouse gas.

DEQ has determined the use of the default data provides a reasonable representation of the greenhouse gas inventory for the various sectors of the state, and the estimated total annual greenhouse gas inventory by year. The SIT data from EPA is currently only updated through the year 2021, as it takes several years to validate and make new data available within revised modules. DEQ maintains a copy of the output results of the SIT.

DEQ has determined that the use of the default data provides a reasonable representation of the GHG inventory for all of the state sectors, and an estimated total annual GHG inventory by year. At present, Montana accounts for 51.04 million metric tons of CO2e based on the EPA SIT for the year 2022. This project may contribute up to 24,075 metric tons per year of CO2e from both Blaine County #3 and Bear Paw Compressor Stations combined. The estimated emission of 24,075 metric tons of CO2e from this project would contribute 0.047% of Montana’s annual CO2e emissions.

GHG emissions that would be emitted as a result of the proposed activities would add to GHG emissions from other sources. The No Action Alternative would contribute less than the Proposed Action Alternative of GHG emissions.

### Description of Alternatives

No Action Alternative: In addition to the proposed action, DEQ must also consider a "no action" alternative. The "no action" alternative would deny the approval of NWE’s Bear Paw Compressor Station The applicant would lack the authority to conduct the proposed activity. Any potential impacts that would result from the proposed action would not occur. The “no action” alternative forms the baseline from which the impacts of the proposed action can be measured.

If the applicant demonstrates compliance with all applicable rules and regulations required for approval, the “no action” alternative would not be appropriate.

Other Reasonable Alternative(s): Describe any other alternatives that were considered.

### Consultation

DEQ engaged in internal and external efforts to identify substantive issues and/or concerns related to the proposed project. Internal scoping consisted of internal review of the environmental assessment document by DEQ staff. External scoping efforts also included queries to the following websites/databases/personnel.

### Public Involvement

The public comment for this permit action was from July 13, 2026 to July 28, 2206. NWE has publicly notified their intent to submit the application in the Havre Daily News on May 20, 2026. No public comment on this environmental assessment was received.

### Significance of Potential Impacts and Need for Further Analysis

When determining whether the preparation of an environmental impact statement is needed, DEQ is required to consider the seven significance criteria set forth in ARM 17.4.608, which are as follows:

- The severity, duration, geographic extent, and frequency of the occurrence of the impact;

- The probability that the impact would occur if the proposed action occurs; or conversely, reasonable assurance in keeping with the potential severity of an impact that the impact would not occur;

- Growth-inducing or growth-inhibiting aspects of the impact, including the relationship or contribution of the impact to cumulative impacts – identify the parameters of the proposed action;

- The quantity and quality of each environmental resource or value that would be affected, including the uniqueness and fragility of those resources and values;

- The importance to the state and to society of each environmental resource or value that would be affected;

- Any precedent that would be set as a result of an impact of the proposed action that would commit the department to future actions with significant impacts or a decision in principle about such future actions; and

- Potential conflict with local, state, or federal laws, requirements, or formal plans.

### Conclusions and Findings

DEQ finds that this action results in negligible impacts to air quality and GHG emissions in Blaine County, Montana.

No significant adverse impacts would be expected because of the proposed project. As noted throughout draft EA, the severity, duration, geographic extent and frequency of the occurrence of impacts associated with the proposed air quality project would be limited. The proposed action would result in the construction and operation of up to two (2) 1,900 bhp engines and one (1) 291 bhp emergency generator.

The applicant proposed to combust pipeline natural gas to power the facility which itself increases natural gas pipeline compression for efficient natural gas transmission.

As discussed in this EA, DEQ has not identified any significant impacts associated with the proposed actions for any environmental resources, DEQ does not believe that the activities proposed by the applicant would have growth-inducing or growth inhibiting aspects, or contributions to cumulative impacts. The proposed site does not appear to contain known or unique fragile resources.

There are no unique or fragile resources in the project area. There would be negligible and minor impacts to the view-shed aesthetics as the engines would be visible to a very few residences. Employees at the operation and at nearby oil and gas operations would see and hear the engine operations when in the immediate area.

Demands on environmental resources of land and water would be minor. However, environmental resources for air and energy would be significant. The compressor station will be powered by the natural gas that it is compressing and transmitting further down the pipeline.

Impacts on human health and safety would not be significant as access to the facility will be closed to the public and the site in private land.

As discussed in this EA, DEQ has not identified any significant impacts associated with he proposed activities on any environmental resource.

Issuance of MAQP #5358-00 to the applicant does not set any precedent that commits DEQ to future actions with significant impacts or a decision in principle about such future actions. If the applicant submits another modification or proposes to amend the permit, DEQ is not committed to issuing those revisions.

Issuance of the permit to the applicant does not set a precedent for DEQ’s review of other applications for permits, including the level of environmental review. The level of environmental review decision is made based on the case-specific considerations of the criteria set forth in ARM 17.4.608

DEQ would conduct an environmental review, or any subsequent permit modification sought by the applicants pursuant to MEPA. DEQ would make a permitting decision based on the criteria set forth in the Clean Air Act of Montana.

Finally, DEQ does not believe that proposed air quality permitting action by the applicant would have any growth inducing or growth inhibiting impacts that would conflict with any local, state, or federal laws, requirements or formal plans.

Based on consideration criteria set forth in ARM 17.4.608 no significant adverse impacts to the affected human environments would be expected because of the proposed project. Therefore, preparation of an Environmental Impact Statement or EIS is not required, and the draft EA is deemed the appropriate level of environmental review pursuant to MEPA.

## Preparation

Environmental Assessment and Significance Determination Prepared By:

Barry Pemberton

Air Quality Scientist

Environmental Assessment Reviewed By:

Craig Henrikson, P.E.

Air Quality Engineer

Approved By:

July 30, 2026

Eric Merchant, Supervisor 			      Date

Air Quality Permitting Services Section

Air Quality Bureau

Air, Energy and Mining Division

Department of Environmental Quality

## References

Include all reference documents and websites used in this EA. These reference materials can be internal documents like fact sheets and circulars, or external materials like government databases, local databases, national studies, etc.

Your citations can be in any style you like (MLA, APA, Chicago, etc.), however, they must clearly tell readers what you referenced and where they can access that information (via link, records request, etc.)

USDA Soil Survey https://websoilsurvey.nrcs.usda.gov/app/WebSoilSurvey.aspx

State Historical Preservation Office Report May 21, 2026

Montana Natural Heritage Program. Environmental Summary Export June 17, 2026

Public School Closing www.publicschoolreview.com

5358-00 NorthWestern Energy Bear Paw Compressor Application

NorthWestern Energy Correspondence for Single Source Determination

NorthWestern Energy Sage Grouse Stewardship Correspondence

2719-10 and 2719-07 Havre Pipeline Company Blaine County #3 Compressor Station Applications

2025 Havre Pipeline Company Annual Report to the Public Service Commission

Blaine County Government Website https://blainecounty-mt.gov/