# MONTANA DEPARTMENT OF ENVIRONMENTAL QUALITY

OPERATING PERMIT TECHNICAL REVIEW DOCUMENT

Permitting and Compliance Division

1520 E. Sixth Avenue

P.O. Box 200901

Helena, Montana 59620-0901

WBI Energy Transmission, Inc.

Hathaway Compressor Station

2010 Montana Avenue

Glendive, MT 59330

NE¼ of the SE¼ of Section 34, Township 7 North, Range 45 East in Custer County, Montana

The following table summarizes the air quality programs testing, monitoring, and reporting requirements applicable to this facility.

| # Facility Compliance Requirements | Yes | No | Comments |
| --- | --- | --- | --- |
| Source Tests Required | X |  |  |
| Ambient Monitoring Required |  | X |  |
| COMS Required |  | X |  |
| CEMS Required |  | X |  |
| Schedule of Compliance Required |  | X |  |
| Annual Compliance Certification and Semiannual Reporting Required | X |  | As applicable |
| Monthly Reporting Required |  | X |  |
| Quarterly Reporting Required |  | X |  |
| # Applicable Air Quality Programs |  |  |  |
| ARM Subchapter 7 Montana Air Quality Permit (MAQP) | X |  | MAQP #1628-04 |
| New Source Performance Standards (NSPS) | X |  | 40 CFR 60, Subpart GG |
| National Emission Standards for Hazardous Air Pollutants (NESHAPS) |  | X | Except for 40 CFR 61, Subpart M |
| Maximum Achievable Control Technology (MACT) |  | X |  |
| Major New Source Review (NSR) includes Prevention of Significant Deterioration (PSD) and/or Non-attainment Area (NAA) NSR |  | X |  |
| Prevention of Significant Deterioration (PSD) |  | X |  |
| Risk Management Plan Required (RMP) |  | X |  |
| Acid Rain Title IV |  | X |  |
| Compliance Assurance Monitoring (CAM) |  | X |  |
| State Implementation Plan (SIP) | X |  | General SIP |

TABLE OF CONTENTS

SECTION I.   GENERAL INFORMATION	4

A.	Purpose	4

B.	Facility Location	4

C.	Facility Background Information	4

D.	Current Permit Action	6

E.	Taking and Damaging Analysis	7

F.	Compliance Designation	7

SECTION II.   SUMMARY OF EMISSION UNITS	8

A.	Facility Process Description	8

B.	Emission Units and Pollution Control Device Identification	8

C.	Categorically Insignificant Sources/Activities	8

SECTION III.   PERMIT CONDITIONS	9

A.	Emission Limits and Standards	9

B.	Monitoring Requirements	9

C.	Test Methods and Procedures	9

D.	Recordkeeping Requirements	10

E.	Reporting Requirements	10

F.	Public Notice	10

G.	Draft Permit Comments	11

SECTION IV.   NON-APPLICABLE REQUIREMENT ANALYSIS	12

SECTION V.   FUTURE PERMIT CONSIDERATIONS	13

A.	MACT Standards (Subpart 63)	13

B.	NESHAP Standards (Subpart 61)	13

C.	NSPS Standards	13

D.	Risk Management Plan	13

E.	Compliance Assurance Monitoring (CAM) Applicability	13

F.	PSD and Title V Greenhouse Gas Tailoring Rule	13

# SECTION I.   GENERAL INFORMATION

## Purpose

This document establishes the basis for the decisions made regarding the applicable requirements, monitoring plan, and compliance status of emission units affected by the operating permit proposed for this facility.  The document is intended for reference during review of the proposed permit by the Environmental Protection Agency (EPA) and the public.  It is also intended to provide background information not included in the operating permit and to document issues that may become important during modifications or renewals of the permit.  Conclusions in this document are based on information provided in the original application submitted on June 12, 1996, the significant modification application submitted on October 17, 2002, the renewal applications submitted on January 30, 2003, January 31, 2008, and November 4, 2013, correspondence received December 10, 2012, and October 17, 2013, and the renewal application submitted on September 30, 2019. As required, a renewal application was submitted on August 1, 2025.

## Facility Location

WBI Energy Transmission, Inc. (WBI) owns and operates a natural gas compressor station located in the Northeast ¼ of the Southeast ¼ of Section 34, Township 7 North, Range 45 East in Custer County, Montana.  The facility is known as the Hathaway Compressor Station.  Custer County is designated as an Unclassifiable/Attainment area for National Ambient Air Quality Standards (NAAQS) for all criteria pollutants.  The facility is located approximately 12 miles southwest of Miles City, Montana.

## Facility Background Information

Montana Air Quality Permit (MAQP)

On August 20, 1981, the Department of Environmental Quality (DEQ) received an application from Montana Dakota Utilities (MDU) for the operation of two 3,370-Horsepower (hp) natural gas compressor engines to be located in Custer County, Montana.  The permit application was assigned MAQP #1628-00.  MAQP#1628-00 became final on October 17, 1981.

On November 23, 1992, DEQ received a request from MDU and Williston Basin Interstate Pipeline Company (WBIPC) to modify Permit #1628-00.  The modification transferred the permit from MDU to WBIPC.  MAQP #1628-01 became final on January 22, 1993.  MAQP #1628-01 replaced MAQP #1628-00.

On February 26, 1993, DEQ received a letter from WBIPC requesting a modification to Permit #1628-01.  The modification correctly identified each of the two 3,730-hp units as turbines rather than reciprocating engines.  MAQP #1628-02 became final on May 7, 1993.  MAQP #1628-02 replaced MAQP #1628-01.

On October 17, 2002, DEQ received an application from WBIPC requesting a modification to Permit #1628-02.  The modification replaced the existing 3,730-hp Solar Centaur turbine (Unit #02) with a refurbished 3,730-hp Solar Centaur turbine (new Unit #02).  In addition, emission limits and testing requirements were added to the permit for the refurbished 3,730-hp Solar Centaur turbine (Unit #02) per the Administrative Rules of Montana (ARM) 17.8.715, as well as the existing 3,370-hp Solar Centaur turbine (Unit #01) as requested by WBIPC.  Further, the permit was updated to reflect current DEQ permit format and permit language.  On December 18, 2002, MAQP #1628-03 replaced MAQP #1628-02.

On December 4, 2012, the Montana Department of Environmental Quality – Air Resources Management Bureau (DEQ) received correspondence from WBI as notification of a change in company name from Williston Basin Interstate Pipeline Company to WBI Energy Transmission, Inc.  This permit action reflected this change in company name as well as updated the MAQP to reflect current DEQ format, rule references, and language. On February 12, 2013, MAQP #1628-04 replaced MAQP #1628-03.

Title V Operating Permit

On June 12, 1996, DEQ received an operating permit application for the Hathaway Station.  The permit application was deemed administratively complete on July 12, 1996, and the application was deemed technically complete on August 12, 1996.  Operating Permit #OP1628-00 became final and effective on September 9, 1998.

On February 4, 1999, DEQ received a letter from WBIPC requesting an administrative amendment to Operating Permit #OP1628-00.  WBIPC requested that DEQ remove the 40 CFR 60, Subpart GG, nitrogen oxides (NOx) standard and corresponding compliance demonstration, recordkeeping and reporting requirements from the Title V permit.  WBIPC stated that because construction commenced prior to October 3, 1982, the NOx requirement is not applicable to the Hathaway Station.  DEQ agreed with WBIPC and performed an administrative amendment to correct the error.  However, the permit number did not change in the permit; but the permit was saved as Operating Permit #OP1628-01 in DEQ’s electronic files.  Therefore, the next permit action will increase the permit number from Operating Permit #OP1628-00 to Operating Permit #OP1628-02.  On February 10, 1999, DEQ amended Operating Permit #OP1628-00.

On October 17, 2002, DEQ received an application from WBIPC requesting a modification to MAQP #1628-02, as well as a modification to Operating Permit #OP1628-00.  WBIPC requested DEQ to significantly modify Operating Permit #OP1628-00 to include the changes at the facility that were incorporated into the MAQP #1628-03.  In addition, on January 30, 2003, DEQ received a Title V Operating Permit Renewal Application from WBIPC.  The application notified DEQ that the only changes that have been made at the facility, since Operating Permit #OP1628-00 was issued, were the changes requested in the significant modification application submitted on October 17, 2002.

The next permit action replaced an existing 3,730-hp Solar Centaur turbine with a refurbished 3,730-hp Solar Centaur turbine.  In addition, emission limits and testing requirements were added to the permit for the refurbished 3,730-hp Solar Centaur turbine (EU #02) per ARM 17.8.752, as well as the existing 3,370-hp Solar Centaur turbine (EU #01) as requested by WBIPC.  Further, the oil and diesel fuel tanks identified as EU #03 in Operating Permit #OP1628-00, were removed from the permit and added to the insignificant emitting unit list.  The permit was a result of the significant modification application of October 17, 2002, as well as the renewal application of January 29, 2003.  Operating Permit #OP1628-02 replaced Operating Permit #OP1628-00.

On January 31, 2008, DEQ received a Title V operating permit renewal application from WBIPC.  Since both of the turbine units had undergone recent refurbishment (2002 for EU #02 and 2005 for EU #01), DEQ requested WBIPC to confirm that the units have not been “constructed, reconstructed, or modified” since 1981.  WBIPC reviewed the activities and costs associated with the recent refurbishment of each turbine and concluded the units have undergone routine maintenance and have not exceeded the 50% cost threshold.  Therefore, the facility remained subject to 40 CFR 60, Subpart GG but exempted from the NOx requirements of 40 CFR 60.332(a), as described under the exemption 40 CFR 60.332(e).  The NOx and carbon monoxide (CO) testing requirements for both units were modified to reflect the 5-year testing requirements in MAQP #1628-03.  In addition, the semi-annual portable analyzer testing requirements were modified to reflect the testing waiver granted to WBIPC if the turbines operated less than 500 hours during any semi-annual period (January 1 to June 30 or July 1 to December 31).  The waiver was granted by DEQ in a letter dated October 13, 2006.  Operating Permit #OP1628-03 replaced Operating Permit #OP1628-02.

On December 10, 2012, DEQ received correspondence from WBI as notification of a change in company name from Williston Basin Interstate Pipeline Company to WBI Energy Transmission, Inc.  Operating Permit #OP1628-04 replaced Operating Permit #OP1628-03.

On October 17, 2013, DEQ received a letter from WBI requesting a Responsible Official change in which Mr. Marc Dempewolf replaces Mr. Scott Fradenburgh.  Mr. Fradenburgh is now considered the Alternate Responsible Official for WBI facilities in the State of Montana.  As such, Operating Permit #OP1628-05 replaces Operating Permit #OP1628-04.

On November 4, 2013, DEQ received a renewal application for Title V Operating Permit #OP1628-05 from WBI. Operating Permit #OP1628-06 replaced Operating Permit # OP1628-05.

On September 30, 2019, DEQ received a renewal application for Title V operating permit #OP1628-06 from WBI. Operating Permit #OP1628-07 replaced Operating Permit # OP1628-06.

## Current Permit Action

On August 1, 2025, DEQ received a renewal application for Title V operating permit #OP1628-07 from WBI. DEQ updated the contact list, merged insignificant Misc. Boilers and Heaters DEQ updated language for Emergency Provisions in the TRD and replaced language referring to the “Department” with DEQ.   Operating Permit #OP1628-08 replaces Operating Permit # OP1628-07.

In July 2023, EPA rescinded the emergency affirmative defense in its Title V regulations. In response, Montana was required to remove the affected provisions from their operating permit program rules and from affected title V operating permits at renewal or during permit revisions. Subsequently, in September 2025, the D.C. Circuit Court of Appeals reinstated the Clean Air Act emergency affirmative defense, reversing the EPA's 2023 rescission of the rule that provides a defense against liability for excess emissions during sudden, unforeseeable emergencies. EPA will need to add the affirmative defense provisions back into its Title V, Part 70 regulations. If this reinstatement does occur, Montana will also add the affected provisions back into its Title V operating permit program and maintain the affected regulations in Title V operating permits.

## Taking and Damaging Analysis

HB 311, the Montana Private Property Assessment Act, requires analysis of every proposed state agency administrative rule, policy, permit condition or permit denial, pertaining to an environmental matter, to determine whether the state action constitutes a taking or damaging of private real property that requires compensation under the Montana or U.S. Constitution.  As part of issuing an operating permit, DEQ is required to complete a Taking and Damaging Checklist.  As required by 2-10-101 through 2-10-105, MCA, DEQ conducted the following private property taking and damaging assessment.

| YES | NO |  |
| --- | --- | --- |
| X |  | 1. Does the action pertain to land or water management or environmental regulation affecting private real property or water rights? |
|  | X | 2. Does the action result in either a permanent or indefinite physical occupation of private property? |
|  | X | 3. Does the action deny a fundamental attribute of ownership? (ex.: right to exclude others, disposal of property) |
|  | X | 4. Does the action deprive the owner of all economically viable uses of the property? |
|  | X | 5. Does the action require a property owner to dedicate a portion of property or to grant an easement? [If no, go to (6)]. |
|  |  | 5a. Is there a reasonable, specific connection between the government requirement and legitimate state interests? |
|  |  | 5b. Is the government requirement roughly proportional to the impact of the proposed use of the property? |
|  | X | 6. Does the action have a severe impact on the value of the property? (consider economic impact, investment-backed expectations, character of government action) |
|  | X | 7. Does the action damage the property by causing some physical disturbance with respect to the property in excess of that sustained by the public generally? |
|  | X | 7a. Is the impact of government action direct, peculiar, and significant? |
|  | X | 7b. Has government action resulted in the property becoming practically inaccessible, waterlogged or flooded? |
|  | X | 7c. Has government action lowered property values by more than 30% and necessitated the physical taking of adjacent property or property across a public way from the property in question? |
|  | X | Takings or damaging implications? (Taking or damaging implications exist if YES is checked in response to question 1 and also to any one or more of the following questions: 2, 3, 4, 6, 7a, 7b, 7c; or if NO is checked in response to questions 5a or 5b; the shaded areas) |

Based on this analysis, DEQ determined there are no taking or damaging implications associated with this permit action.

## Compliance Designation

DEQ conducted an inspection and a Full Compliance Evaluation, with the report sent March 23, 2026.  At the time of this report, the facility appeared to be in compliance with the applicable permit conditions.

# SECTION II.   SUMMARY OF EMISSION UNITS

## Facility Process Description

WBI’s Hathaway Compressor Station is a natural gas pipeline booster station that utilizes two natural gas turbines to drive compressors that boost the pressure in the natural gas pipeline to allow the transmission of natural gas.  The Hathaway Compressor Station transmits natural gas either east to WBI’s Cabin Creek Compressor Station or west to WBI’s Hardin Compressor Station.

## Emission Units and Pollution Control Device Identification

| Emissions Unit I.D. | Description | Installation Date | Last Refurbishment | NSPS Applicability | Pollution Control Device |
| --- | --- | --- | --- | --- | --- |
| EU001 | 3,730-hp Solar Centaur Turbine | 12/18/81 | 2005 | Subpart GG* | None |
| EU002 | 3,730-hp Solar Centaur Turbine | 9/4/81 | 2002 | Subpart GG* | None |

Note:  The units are subject to 40 CFR 60, Subpart GG, but are exempt from the NOx requirements in 40 CFR 60.332(a) due to an exemption under 40 CFR 60.332(e).

## Categorically Insignificant Sources/Activities

The Administrative Rules of Montana (ARM) 17.8.1201(22)(a) defines an insignificant emissions unit as one that emits less than five tons per year of any regulated pollutant, has the potential to emit less than 500 pounds per year of lead or any hazardous air pollutant, and is not regulated by any applicable requirement other than a generally applicable requirement.  The insignificant emitting units identified by WBI in their renewal application are summarized in the following table.

| Emissions Unit ID | Description |
| --- | --- |
| Misc 1 | Misc. Boilers and Heaters |
| FUG | Fugitive Emissions from valves, flanges, open ended lines, etc. |
| IEU015 | Fugitive Emissions from in plant vehicle traffic |

# SECTION III.   PERMIT CONDITIONS

## Emission Limits and Standards

Each of the two 3,730-hp Solar Centaur turbines is limited to 16.45 pound per hour (lb/hr) for NOx, 24.67 lb/hr for CO, and 8.22 lb/hr for volatile organic compounds (VOC).  The emission limits are based on Best Available Control Technology (BACT) determinations that were established by DEQ.  In addition, the natural gas is limited to a sulfur content of 0.8% by weight and a sulfur dioxide (SO2) content of 0.015 % by volume at 15% oxygen (O2) as required under 40 CFR 60, Subpart GG.

## Monitoring Requirements

ARM 17.8.1212(1) requires that all monitoring and analysis procedures or test methods required under applicable requirements are contained in operating permits.  In addition, when the applicable requirement does not require periodic testing or monitoring, periodic monitoring must be prescribed that is sufficient to yield reliable data from the relevant time period that is representative of the source's compliance with the permit.

The requirements for testing, monitoring, recordkeeping, reporting, and compliance certification sufficient to assure compliance do not require the permit to impose the same level of rigor for all emission units.  Furthermore, they do not require extensive testing or monitoring to assure compliance with the applicable requirements for emission units that do not have significant potential to violate emission limitations or other requirements under normal operating conditions.  When compliance with the underlying applicable requirement for an insignificant emissions unit is not threatened by lack of regular monitoring and when periodic testing or monitoring is not otherwise required by the applicable requirement, the status quo (i.e., no monitoring) will meet the requirements of ARM 17.8.1212(1).  Therefore, the permit does not include monitoring for insignificant emission units.

The permit includes periodic monitoring or recordkeeping for each applicable requirement.  The information obtained from the monitoring and recordkeeping will be used by the permittee to periodically certify compliance with the emission limits and standards.  However, DEQ may request additional testing to determine compliance with the emission limits and standards.

## Test Methods and Procedures

Compliance with the opacity, particulate from fuel combustion, sulfur compounds in fuel (gaseous), and VOC limitations in the permit may be demonstrated by burning pipeline quality natural gas on a continuous basis.  In addition, WBI is required to conform with the requirements in 40 CFR 60, Subpart GG to demonstrate that the sulfur content of the natural gas is less than 0.8% by weight and the SO2 content of the natural gas is less than 0.015 % by volume at 15% O2 on a dry basis.

MAQP #1628-04 requires WBI to test each of the two 3,730-hp Solar Centaur turbines for NOx and CO, concurrently, on an every 5-year basis to demonstrate compliance with the emission limitations in the permit.  The permit demands that the tests be performed according to the EPA methods in Appendix A of 40 CFR 60.  This operating permit also contains requirements for semi-annual testing with a portable analyzer for each of the two 3,730-hp Solar Centaur turbines.  This permit stipulates that the portable analyzer shall be capable of achieving performance specifications equivalent to the traditional test methods in 40 CFR 60, Appendix A or shall be capable of meeting the requirements of EPA Conditional Test Method 022 for the “Determination of Nitric Oxide, Nitrogen Dioxide and NOx from Stationary Sources by Electrochemical Analyzer.”

In addition, in a letter dated October 12, 2006, DEQ approved a testing waiver if a turbine operates less than 500 hours in a semiannual period (January 1 to June 30 or July 1 to December 31).  The letter specifies that the semiannual test will still be performed regardless of hours of operation if any changes are made that may affect emissions.  It also requires that hours of operation be documented and records kept for at least 5 years.

WBI may use another testing procedure as approved in advance by DEQ.  All compliance tests must be conducted in accordance with the Montana Source Test Protocol and Procedures Manual (ARM 17.8.106).

WBI will then convert the NOx and CO emissions test results from a “part per million (ppm)” value to a “lb/hr” number.  Stack gas flow rates shall be determined using EPA Test Methods in 40 CFR 60, Appendix A in order to monitor compliance with the emissions limitations in the permit.

DEQ will use the portable analyzer testing results as a direct measure of compliance.  The operating permit may not require testing for all sources if routine monitoring is used to determine compliance, but DEQ has the authority to require testing if deemed necessary to determine compliance with an emission limit or standard.  In addition, the permittee may elect to voluntarily conduct compliance testing to confirm its compliance status.

## Recordkeeping Requirements

The permittee is required to keep all records listed in the operating permit as a permanent business record for at least five years following the date of the generation of the record.

## Reporting Requirements

Reporting requirements are included in the permit for each emissions unit and Section V of the operating permit "General Conditions" explains the reporting requirements.  However, the permittee is required to submit semi-annual monitoring reports to DEQ and to annually certify compliance with the applicable requirements contained in the permit.  The reports must include a list of all emission limit and monitoring deviations, the reason for any deviation, and the corrective action taken as a result of any deviation.

## Public Notice

In accordance with ARM 17.8.1232, a public notice was published in the Miles City Star newspaper on or before May 13, 2026.  DEQ provided a 30-day public comment period on the draft operating permit from May 13, 2026, to June 12, 2026.  ARM 17.8.1232 requires DEQ to keep a record of both comments and issues raised during the public participation process.  The comments and issues received by June 12, 2026, will be summarized, along with DEQ's responses, in the following table.  All comments received during the public comment period will be promptly forwarded to WBI so they may have an opportunity to respond to these comments as well.

### Summary of Public Comments

| ### Summary of Public Comments | ### Summary of Public Comments | ### Summary of Public Comments |
| --- | --- | --- |

## Draft Permit Comments

Summary of Permittee Comments

| Permit Reference | Permittee Comment | Department Response |
| --- | --- | --- |

### Summary of EPA Comments

| Permit Reference | EPA Comment | Department Response |
| --- | --- | --- |

# SECTION IV.   NON-APPLICABLE REQUIREMENT ANALYSIS

Section IV of the operating permit “Non-Applicable Requirements” contains the requirements that DEQ determined were non-applicable.  The following table summarizes the requirements that WBI identified as non-applicable and contains the reasons that DEQ did not include these requirements as non-applicable in the permit.

| Applicable Requirement | Reason Not Included in Permit |
| --- | --- |
| 40 CFR 61, Subpart M National Emissions Standards for Hazardous Air Pollutants - Asbestos | This is a federal regulation that has specific procedural requirements that may become relevant to the major source during the permit term. |

# SECTION V.   FUTURE PERMIT CONSIDERATIONS

## MACT Standards (Subpart 63)

As of the issuance date of Operating Permit #OP1628-08, DEQ is unaware of any future MACT Standards that may be promulgated that will affect this facility.

## NESHAP Standards (Subpart 61)

As of the issuance date of Operating Permit #OP1628-08, DEQ is unaware of any future NESHAP Standards that may be promulgated that will affect this facility.

## NSPS Standards

As of the issuance date of Operating Permit #OP1628-08, DEQ is unaware of any future NSPS Standards that may be promulgated that will affect this facility.

## Risk Management Plan

As of the issuance date of Operating Permit #OP1628-08, this facility does not exceed the minimum threshold quantities for any regulated substance listed in 40 CFR 68.115 for any facility process.  Consequently, this facility is not required to submit a Risk Management Plan.

If a facility has more than a threshold quantity of a regulated substance in a process, the facility must comply with 40 CFR 68 requirements no later than June 21, 1999; three years after the date on which a regulated substance is first listed under 40 CFR 68.130; or the date on which a regulated substance is first present in more than a threshold quantity in a process, whichever is later.

## Compliance Assurance Monitoring (CAM) Applicability

An emitting unit located at a Title V facility that meets the following criteria listed in ARM 17.8.1503 is subject to Subchapter 15 and must develop a CAM Plan for that unit:

- The emitting unit is subject to an emission limitation or standard for the applicable regulated air pollutant (unless the limitation or standard is exempt under ARM 17.8.1503(2))

- The emitting unit uses a control device to achieve compliance with such limit; and

- The emitting unit has potential pre-control device emissions of the applicable regulated air pollutant that are greater than major source thresholds.

WBI does not currently have any emitting units that meet all the applicability criteria in ARM 17.8.1503 and is therefore not currently required to develop a CAM Plan.

## PSD and Title V Greenhouse Gas Tailoring Rule

On May 7, 2010, EPA published the “light duty vehicle rule” (Docket # EPA-HQ-OAR- 2009-0472, 75 FR 25324) controlling greenhouse gas (GHG) emissions from mobile sources, whereby GHG became a pollutant subject to regulation under the Federal and Montana Clean Air Act(s).  On June 3, 2010, EPA promulgated the GHG “Tailoring Rule” (Docket # EPA-HQ-OAR-2009-0517, 75 FR 31514) which modified 40 CFR Parts 51, 52, 70, and 71 to specify which facilities are subject to GHG permitting requirements and when such facilities become subject to regulation for GHG under the PSD and Title V programs.

Under the Tailoring Rule, any PSD action (either a new major stationary source or a major modification at a major stationary source) taken for a pollutant or pollutants other than GHG that would become final on or after January 2, 2011, would be subject to PSD permitting requirements for GHG if the GHG increases associated with that action were at or above 75,000 TPY of carbon dioxide equivalent (CO2e) and greater than 0 TPY on a mass basis.  Similarly, if such action were taken, any resulting requirements would be subject to inclusion in the Title V Operating Permit.  Facilities which hold Title V permits due to criteria pollutant emissions over 100 TPY would need to incorporate any GHG applicable requirements into their operating permits for any Title V action that would have a final decision occurring on or after January 2, 2011.

Starting on July 1, 2011, PSD permitting requirements would be triggered for modifications that were determined to be major under PSD based on GHG emissions alone, even if no other pollutant triggered a major modification.  In addition, sources that are not considered PSD major sources based on criteria pollutant emissions would become subject to PSD review if their facility-wide potential emissions equaled or exceeded 100,000 TPY of CO2e and 100 or 250 TPY of GHG on a mass basis depending on their listed status in ARM 17.8.801(22) and they undertook a permitting action with increases of 75,000 TPY or more of CO2e and greater than 0 TPY of GHG on a mass basis. With respect to Title V, sources not currently holding a Title V permit that have potential facility-wide emissions equal to or exceeding 100,000 TPY of CO2e and 100 TPY of GHG on a mass basis would be required to obtain a Title V Operating Permit.

The Supreme Court of the United States (SCOTUS), in its Utility Air Regulatory Group v. EPA decision on June 23, 2014, ruled that the Clean Air Act neither compels nor permits EPA to require a source to obtain a PSD or Title V permit on the sole basis of its potential emissions of GHG.  SCOTUS also ruled that EPA lacked the authority to tailor the Clean Air Act’s unambiguous numerical thresholds of 100 or 250 TPY to accommodate a CO2e threshold of 100,000 TPY.  SCOTUS upheld that EPA reasonably interpreted the Clean Air Act to require sources that would need PSD permits based on their emission of conventional pollutants to comply with BACT for GHG.  As such, the Tailoring Rule has been rendered invalid and sources cannot become subject to PSD or Title V regulations based on GHG emissions alone.  Sources that must undergo PSD permitting due to pollutant emissions other than PSD may still be required to comply with BACT for GHG emissions.