# DRAFT ENVIRONMENTAL ASSESSMENT

August 14, 2026

Waste Management and Remediation Division

Montana Department of Environmental Quality

PROJECT/SITE NAME: Labatte Farms

APPLICANT/COMPANY NAME: Under Pressure Septic Pumping LLC

LICENSE NUMBER: S-1175

LOCATION: Section 14, Township 28 North, Range 56 East

COUNTY: Roosevelt

PROPERTY OWNERSHIP: PRIVATE

Table of Contents

DRAFT ENVIRONMENTAL ASSESSMENT	1

1.	Overview of Proposed Action	3

1.1	Authorizing Action	3

1.2	Description of DEQ Regulatory Oversight	3

1.3	Proposed Action	3

1.4	Purpose, Need, and Benefits	5

1.5	Amount and Extent of Septage Application	6

1.6	Other Governmental Agencies and Programs with Jurisdiction	6

2.	Evaluation of Affected Environment And Impact by Resource	8

2.1	Geology and Soil Quality, Stability and Moisture	8

2.2	Water Quality, Quantity, And Distribution	9

2.3	Air Quality	10

2.4	Vegetation Cover, Quantity, and Quality	10

2.5	Terrestrial, Avian, and Aquatic Life and Habitats	11

2.6	History, Culture, and Archaeological Uniqueness	11

2.7	Demands on Environmental Resources of Land, Water, Air, or Energy	12

2.8	Human Health and Safety	12

2.9	Aesthetics	13

2.10	Socioeconomics	13

2.11	Private Property Impacts	14

2.12	Greenhouse Gas Assessment	14

3.	Description of Alternatives	15

4.	Consultation	15

5.	Public Involvement	15

6.	Significance of Potential Impacts and Need for Further Analysis	15

7.	Conclusions and Findings	16

8.	Preparation	16

9.	References	18

## Overview of Proposed Action

### 1.1	Authorizing Action

Under the Montana Environmental Policy Act (MEPA), Montana agencies are required to prepare an environmental review for state actions that may have an impact on the Montana environment. The  proposed action is considered to be a state action that may have an impact on the Montana environment and, therefore, the Department of Environmental Quality (DEQ) must prepare an environmental review. This EA will examine the proposed action and alternatives to the proposed action and disclose potential and proximate impacts that may result from the proposed and alternative actions. DEQ will determine the need for additional environmental review based on consideration of the criteria set forth in Administrative Rules of Montana (ARM) 17.4.608.

### 1.2	Description of DEQ Regulatory Oversight

DEQ implements the Septage Disposal and Licensure laws at Title 75, chapter 10, part 12, Montana Code Annotated (MCA), overseeing the licensure of persons engaged in the business of cleaning cesspools, septic tanks, and privies or disposing of septage. Included in this authority, DEQ oversees the approval and management of septage land application sites. DEQ is also responsible for the regulation of air and water quality under the Clean Air Act of Montana (CAA), §§ 75-2-101, et seq., MCA, maintaining air quality standards that protect human health and safety and the Montana Water Quality Act (WQA), which governs discharges of pollutants to Montana state waters (75-5-101, et seq., MCA). DEQ continuously oversees land application sites for compliance with septage land application rules (ARM 17.50.812).

### 1.3	Proposed Action

DEQ would approve a disposal land application site in Section 14, Township 28 North, Range 56 East, in Roosevelt County, Culbertson, Montana, under the Septage Disposal and Licensure laws for Under Pressure Septic Pumping, LLC, if DEQ has determined that Under Pressure Septic Pumping, LLC met the criteria set forth in Title 75, chapter 10, part 12, MCA, and ARM Title 17, chapter 50, subchapter 8. The proposed action is depicted in Figure 1 below. All information included in this EA is derived from the application, discussions with the applicant, analysis of aerial photography, topographic maps, and other research tools.

Figure 1: General Location of the Proposed Project

Table 1: Summary of Proposed Action

| General Overview | Upon approval of the new land application site, Under Pressure Septic Service would begin land applying septage on the approved site. The land application would include a truck (1999 Freightliner F106) with a 1,000-gallon tank driving into a field, releasing a valve, and allowing septage to be spread onto the field. The truck then drives along the field until the truck is empty. The pumper then tills the septage into the soil to reduce vector attraction. This activity replaces the use of chemical fertilizers as a soil amendment. Setbacks outlined in ARM along with other operations would be met and prevent harm to human health and the physical environment. |
| --- | --- |
| Estimated Disturbance | Septage would be land applied on approximately 20 acres of private land on the Labatte Farm property. |
| Location and Analysis Area | Location: Section 14, Township 28 North, Range 56 East, Roosevelt County, approximately four miles northeast of Culbertson, Montana. Analysis Area: The area being analyzed as part of this environmental review includes the immediate project area (Figure 1), as well as neighboring lands surrounding the analysis area, as reasonably appropriate for the impacts being considered. |

Table 2. The applicant is required to comply with all applicable local, county, state, and federal requirements pertaining to the following resource areas.

| Air Quality | Because of the moisture of the septage, any dust would be minimized by the land application of the septage being disposed across the site. |
| --- | --- |
| Water Quality | To protect surface and groundwater resources, the applicant must not apply pumpings to land within 100 feet of a drinking water supply source, or to apply pumpings to land within 150 feet of any state surface water, including ephemeral or intermittent drainages and wetlands per ARM 17.50.509 (2) and (3). |
| Erosion Control and Sediment Transport | To reduce impacts of erosion, the applicant cannot apply pumpings on land of a slope greater than 6% per ARM 17.50.509 (6). |
| Solid Waste | The applicant would be required to remove litter from the proposed site within six hours of application per ARM 17.50.509 (10). |
| Cultural Resources | In a letter to the applicant, the State Historic Preservation Office (SHPO) stated that as long as there would be no disturbance or alteration to structures over fifty years of age, they feel that there will be no cultural or historic properties affected by this undertaking. They, therefore, feel that a recommendation for a cultural resource inventory is unwarranted at this time. However, should structures need to be altered or if cultural materials are inadvertently discovered during the proposed action, they ask that their office be contacted, and the site investigated. |

Table 3: Cumulative Impacts

| Past Actions | The site has been used for agriculture. This site has never been approved for septage land application previously. |
| --- | --- |
| Present Actions | The site is currently utilized for grass and barley production by the landowner. |
| Related Future Actions | Future actions are unknown at this time. The applicant may submit to DEQ an application to amend the plan for the site at any time, which DEQ would review pursuant to Montana law, 75-10-1202, MCA. Any future actions would be subject to corresponding MEPA review. |

### 1.4	Purpose, Need, and Benefits

DEQ's purpose in conducting this environmental review is to act upon Under Pressure Septic Service’s application for a septic tank pumper license with Labatte Farms as their only disposal site. DEQ’s action on the application is governed by § 75-10-1211, et seq., Montana Code Annotated (MCA) and the Administrative Rules of Montana (ARM) 17.50.803, et seq.  DEQ made a preliminary decision that the application was complete on March 26, 2026.

The applicant’s purpose and need, as expressed to DEQ in seeking this action, is to properly manage land application of septage. Under Pressure Septic Service, LLC’s application was submitted to DEQ under the laws and rules for licensing septic tank pumpers, demonstrating their intent to meet the minimum requirements for the pumping and land application of septage. A licensed land application program recognizes and employs practices that maximize those benefits.

### 1.5	Amount and Extent of Septage Application

Land application of septage must not exceed the Annual Application Rate (AAR), in gallons per acre per year, based on:

1. The nitrogen content of the waste applied at the Site (EPA, 1993); and

2. The crop nitrogen yields for the crop or other vegetation at the Site.

The AAR for septage waste is calculated as follows:

AAR = minimum crop nitrogen requirement (lbs./acre/year)

0.0026 (lbs./gallon)

Twenty (20) acres of the Site would be used to grow grass. The nitrogen requirement for pasture grass is 75 pounds per acre per year based on a conservative yield expectation at the Site (Montana State University). For the grass, the resulting AAR for septage is 28,846.2 gallons per acre per year, which is equal to less than 3/4 inches of liquid applied annually per acre. For comparison, the average annual precipitation in the Culbertson area is 13.5 inches per year.

Land application of septage at the AAR is alternated annually between separate parcels to allow for agronomic crop uptake of the applied nitrogen. Plants can fully utilize nitrogen available from the septage if the volume of septage applied each year does not exceed the AAR. When land application is rotated, one parcel is used every year: for example, if 100 acres are proposed for land application, 50 acres would be used one year and the other 50 acres would be used similarly the next year. In this case, Under Pressure Septic Service would rotate the Site’s acreage each year. The residual soil nutrient levels at each parcel would vary over time. DEQ may periodically monitor the soil for nutrient content to determine compliance with the AAR.

The maximum capacity of the site per year when rotated annually is 288,462 gallons and the proposed amount of septage to be applied is 10,000 gallons per year.

Based on these calculations the Labatte Farms property could treat the proposed volume of waste without exceeding the site AAR each year.

### 1.6	Other Governmental Agencies and Programs with Jurisdiction

The proposed action would be located on private land. All applicable local, state, and federal rules must be adhered to, which may also include other local, state, federal, or tribal agency jurisdiction. Other governmental agencies which may have overlapped, or additional jurisdiction include but may not be limited to: Roosevelt County and the Roosevelt County Health Department.

## Evaluation of Affected Environment And Impact by Resource

The impact analysis will identify and evaluate the direct and secondary impacts to the physical environment and population in the area to be affected by the proposed project. Direct impacts occur at the same time and place as the action that causes the impact. Secondary impacts are a further impact to Montana’s environment that may be stimulated, induced by, or otherwise result from a direct impact of the action (ARM 17.4.603(18)). Where impacts would occur, the impacts will be described in this analysis. When the analysis discloses environmental impacts, these are proximate impacts pursuant to 75-1-201(1)(b)(iv)(A), MCA.

Cumulative impacts are the collective impacts on Montana’s environment within the borders of Montana of the proposed action when considered in conjunction with other past and present actions related to the  proposed action by location and generic type. Related future actions must also be considered when these actions are under concurrent consideration by any state agency through pre-impact statement studies, separate impact statement evaluation, or license processing procedures. The projects identified in Table 1 were analyzed as part of the cumulative impacts assessment for each resource.

The duration is quantified as follows:

- Short-term Impacts (short-term): These are impacts to the environment during the active land application period and shortly thereafter.

- Operation Impacts (long-term): These are impacts to the environment during the operational period.

The intensity of the impacts is measured using the following:

- No impact: There would be no change from current conditions.

- Negligible: An adverse or beneficial effect would occur but would be at the lowest levels of detection.

- Minor: The effect would be noticeable but would be relatively small and would not affect the function or integrity of the resource.

- Moderate: The effect would be easily identifiable and would change the function or integrity of the resource.

- Major: The effect would alter the resource.

### 2.1	Geology and Soil Quality, Stability and Moisture

The proposed site lies roughly four miles northeast of the Missouri river and approximately three miles northeast of the town of Culbertson, Montana. The surficial geology of the analysis area consists of Pleistocene glacial till. The proposed site is entirely on well-draining Cherry silt loam.

Direct Impacts

No impacts to geology would be anticipated because of the  proposed action. The geology of the area remains unaltered or unaffected by agriculture activities. The applicant would be required to not apply pumpings on land with a greater than 6% slope per ARM 17.50.509 (6). A minor positive impact to soils would be anticipated due to the additional moisture and nutrients from land septage application.

Secondary Impacts

No secondary impacts to geology or soils would be expected because of the proposed action.

Cumulative Impacts

No cumulative impacts to geology or soils would be expected because of the proposed action.

### 2.2	Water Quality, Quantity, And Distribution

Annual precipitation in the area is approximately 13.5 inches (USDA National Resource Conservation Service). The location of the site has a slope near zero (0.0%) which means no runoff will occur and all septage would absorb into the soil. The site is located outside the 100-year floodplain. Primary surface water bodies are located more than 150 feet away from the proposed location. Drinking water sources are also located greater than 100 feet from the proposed disposal location. Depth to groundwater in the area ranged between 55 feet to 120 feet below ground surface according to the closest groundwater wells mapped by the Groundwater Information Center (GWIC). The closest well to the proposed application site has a depth-to-water of 62 feet, as recorded at the time of drilling.

The proposed location does not contain any areas designated as wetland habitat by the United States Fish and Wildlife Services (USFWS) (Wetlands Map) or the Montana National Heritage Program (MTNHP) (Map Viewer). The nearest intermittent stream is located approximately 370 feet north of the proposed site as mapped by the USFWS National Wetlands Inventory Map.

Direct Impacts

No impact to surface water would occur as the applicant cannot apply pumpings to land within 100 feet of a drinking water supply source or apply pumpings to land within 150 feet of any state surface water, including ephemeral or intermittent drainages and wetlands per ARM 17.50.509 (2) and (3). There will be no stormwater runoff at the proposed site since slope at the site is less than 6%.

Secondary Impacts

No secondary impacts to water quality would be expected. Stormwater impacting the site during a precipitation event will infiltrate into the subsurface and would not leave the site. This is due to the site having a slope less than 6%. No amounts of sediment can be carried from the site and deposited via stormwater.

Cumulative Impacts

Cumulative impacts on surface-water and groundwater quality would not be expected. The surrounding areas are brushland used for grass and barley production. Animals are allowed to graze on the site 30 days after application of pumpings per ARM 17.50.811. Therefore, interactions between other operations and the proposed action mean no cumulative impacts on surface waters, or groundwater would occur.

### 2.3	Air Quality

According to the Clean Air Act of 1977, any national park that is greater than 6,000 acres and any wilderness area greater than 5,000 acres are considered Class 1 airsheds. Although Class 1 airsheds are managed and regulated by the National Park Service, USFWS, U.S. Forest Service and Native American Tribes, the state may still redesignate areas to be considered Class 1 airsheds to better protect the air quality of a certain area. The nearest Class I airsheds are the Fort Peck Reservation and the Medicine Lake Wilderness. The Fort Peck Reservation is located roughly 7 miles west of the proposed site, and the Medicine Lake Wilderness is roughly 17 miles north of the proposed site (EPA). Septage would be incorporated into the soil surface within six hours of application and dust would be controlled by the moisture being applied to the soil.

Direct Impacts

No impact to air quality would be expected because dust would be controlled through the moisture during septage application.

Secondary Impacts

No secondary impacts to air quality would be expected because of the proposed action.

Cumulative Impacts

No cumulative impacts to air quality would be expected because of the proposed action.

### 2.4	Vegetation Cover, Quantity, and Quality

The MTNHP compiles an online report to classify plant Species of Concern and Potential Concern in the state, employing a standardized ranking system to denote global (range-wide) and state status. Species are assigned numeric ranks ranging from 1 (highest risk, greatest concern) to 5 (demonstrably secure), reflecting the relative degree of risk to the species’ viability, based upon available information. Species of Concern are native taxa that are at-risk due to declining population trends, threats to their habitats, restricted distribution, and/or other factors. Designation as a Montana Species of Concern or Potential Species of Concern is based on the Montana Status Rank and is not a statutory or regulatory classification (MTNHP, 2025). MTNHP was consulted for the proposed project area.  There are no plant species of concern within the analysis area.  Other potential species of concern not observed occurring in the analysis area have medium to no suitability for the site.

The site is currently grassland. There would be no grazing or crop harvesting at the site as stated in the application.

Direct Impacts

Minor direct impacts to the resource are expected as septage is applied and the ground is then tilled to prevent vector attraction and to reduce pathogens. The site is accessed via private road, so no additional vegetation disturbance would occur from vehicle traffic.

Secondary Impacts

Beneficial secondary impacts to the resource could occur from the addition of moisture and nitrogen in the applied septage to the soil. Additional moisture and nitrogen at an agronomic rate would benefit vegetation growth.

Cumulative Impacts

No cumulative impacts to the resource would be expected because of the proposed action.

### 2.5	Terrestrial, Avian, and Aquatic Life and Habitats

Montana Animal Species of Concern are native Montana animals that are considered to be "at risk" due to declining population trends, threats to their habitats, and/or restricted distribution and are reported jointly between the MTNHP and Montana Department of Fish, Wildlife, and Parks (MFWP). Designation as a Montana Species of Concern or Potential Species of Concern is based on the Montana Status Rank and is not a statutory or regulatory classification (MTHP, 2022). The species of concern occurring in the analysis area at the proposed site would be Long-billed Curlew, Chimney Swift, Red-headed woodpecker, and Loggerhead Shrike. The analysis area is also listed as important animal habitat for non-cave bat roosts. The species of concern listed above have medium to low suitability for the site.  Other species of concern with no occurrences in the analysis area have low to no suitability for the site. There are no wetland areas on the site. The site is not located in sage grouse habitat according to the Montana Sage Grouse Habitat Conservation Program.

Direct Impacts

The current land use for the site is grassland, so minor impacts to the resource would be expected as disturbance occurs from the application. Wildlife could encounter litter contained in the applied septage, but this would be mitigated as septic pumpers are required to screen or pick litter from their application sites as stipulated in ARM 17.50.811 Operation and Maintenance Requirements for Land Application or Incorporation of Septage. Further, wildlife tends to stay away from areas from high human or agricultural activities and odors.

Secondary Impacts

No secondary impacts to the resource would be expected because of the proposed action.

Cumulative Impacts

No cumulative impacts to the resource would be expected because of the proposed action.

### 2.6	History, Culture, and Archaeological Uniqueness

The State Historic Preservation Office (SHPO) stated that as long as there would be no disturbance or alteration to structures over fifty years of age, they feel that there will be no cultural or historic properties affected by this undertaking. They, therefore, feel that a recommendation for a cultural resource inventory is unwarranted at this time. However, should structures need to be altered or if cultural materials are inadvertently discovered during the proposed action, they ask that their office be contacted, and the site investigated.

Direct Impacts

No direct impacts on history, culture, or archaeological uniqueness of the property would be expected because of the proposed action.

Secondary Impacts

No secondary impacts to the property would be expected because of the proposed action.

Cumulative Impacts

No cumulative impacts would be expected because of the proposed action.

### 2.7	Demands on Environmental Resources of Land, Water, Air, or Energy

The proposed action would use some fuel for the operation of the septic tank pumper vehicle. However, this fuel usage would not be expected to be an unusual demand on the area’s energy resources. There would be no unusual demands on the land, water, air, or energy. This activity would add nitrogen and moisture to the soil and would replace the use of chemical fertilizers as a soil amendment.

Direct Impacts

Based on the analysis of available data, DEQ does not foresee any unusual demands on land, water, air or energy resulting from this proposed action. Therefore, no direct impacts would be anticipated.

Secondary Impacts

No secondary impacts on environmental resources would be expected because of the proposed action.

Cumulative Impacts

No impacts on environmental resource demands would be expected because of the proposed action.

### 2.8	Human Health and Safety

The applicant would be required to adhere to all applicable state and federal safety laws. The Occupational Safety and Health Administration (OSHA) has developed rules and guidelines to reduce the risks associated with this type of labor. Few, if any, members of the public would be in immediate proximity to the project during operations.

Septic tank waste would be land applied at the proposed site. The waste would be tilled in to prevent vector attraction per ARM 17.50.811(3)(c). No livestock would graze on the site within 30 days of land application per ARM 17.50.811(5)(a).

Direct Impacts

Direct impacts to human health and safety of the proposed action are mitigated by OSHA rules and guidelines. No direct risks to human health and safety are expected from the proposed action. Lehman Septic is required to follow applicable state and federal safety laws. The setback requirements outlined in ARM 17.50.809(1-4) limit any potential impacts to human health from septage land application.

Secondary Impacts

No secondary impacts to human health and safety would be expected from the  proposed action because of the surface water and drinking water supply setback requirements.

Cumulative Impacts

No cumulative impacts to human health and safety would be expected from the  proposed action.

### 2.9	Aesthetics

Minor impacts to aesthetics and noise are expected because of the  proposed action. The analysis area is the Site and the surrounding area within one mile of the Site.

Direct Impacts

The proposed action would not be visible from any state or county roads. Only one truck would access the proposed site at a time. The pumper truck would access the proposed site via a private drive to conduct land application activities. Noise from the truck at the proposed site would resemble noises from agricultural activities currently occurring in the area. Minor visual and auditory impacts could occur intermittently for the duration of the proposed action. The naturally occurring bacteria in the soil would use carbon and other nutrients in the waste as a fuel source. This activity results in the breakdown of waste, which includes odors. Usually, odors are only detected at the time and immediate vicinity (within feet) of the land application activity. Therefore, a temporary olfactory impact could occur, but would be controlled by pH adjustment. Land application could occur daily. Dust caused by the truck driving during the dry season would be reduced by the moisture content of septage.

Secondary Impacts

No secondary impacts to human health and safety would be expected from the  proposed action because of the aesthetics.

Cumulative Impacts

No cumulative impacts to aesthetics would be expected because of the proposed action.

### 2.10	Socioeconomics

In preparing an environmental assessment, ARM 17.4.609(3)(e) directs the agency to evaluate, where appropriate, potential topics related to impacts on the human population in the area to be affect by the proposed action. DEQ has determined that the following topic areas are not anticipated to result in impacts that are potentially significant, and thus no further analysis is required:

- Industrial, Commercial, and Agricultural Activities and Production: The project would not require construction, road work, or have any socioeconomic impact due to the proposed action.

- Quantity and Distribution of Employment: Due to the scale of the proposed action, it is unlikely that employment opportunities would be created as a result of this land application site. The proposed action is not expected to move or eliminate jobs.

- Local and State Tax Base and Revenues: Local, state and federal governments would be responsible for appraising the property, setting tax rates, collecting taxes, etc., from the companies, employees, or landowners benefiting from this operation. Since the proposed site has been used as an agricultural site in the past, it is unlikely that the tax base on the land would be expected to change.

- Demand for Government Services: The location of the land septage application would be in a rural agricultural area. As stated by the applicant in the application, one Freightliner F106 would be used for septage application. This would be consistent with current traffic conditions in the area and is not expected to cause an increase in traffic.

- Locally Adopted Environmental Plans and Goals: The project area would be subject to any plans or rules set forth by Roosevelt County.

- Access to and Quantity of Recreational and Wilderness Activities: The proposed activities would occur on private land. The project would not limit access to wilderness or recreational areas nearby.

- Density and Distribution of Population and Housing: The project would occur on private land. The area has a population of less than 1,000. It is also unlikely that the proposed action would add to the population or require additional housing.

- Social Structures and Mores: DEQ is not aware of any native cultural concerns that would be affected by the proposed activity. This topic is also addressed in Section 2.6 above. It is not anticipated that this project would disrupt native or traditional lifestyles or communities.

- Cultural Uniqueness and Diversity: DEQ is not aware of any native cultural concerns that would be affected by the proposed activity. This topic is also addressed in Section 2.6 above. It is not anticipated that this project would disrupt native or traditional lifestyles or communities.

### 2.11	Private Property Impacts

The proposed action would take place on private land owned by Labatte Farms, the property owner has agreed to allow the proposed action on their land. DEQ’s approval would affect the proposed site as noted in this Draft EA’s impacts analysis. However, license conditions ensure compliance with the rules and statutes in place for septage land application. DEQ’s approval of the proposed action would not have private property-taking or damaging implications.

### 2.12	Greenhouse Gas Assessment

DEQ is required to evaluate greenhouse gas (GHG) emissions for proposed actions that meet the statutory definition of a fossil fuel activity, in accordance with 75-1-211 and 75-1-220, MCA. However, pursuant to 75-1-220(8), this proposed action is excluded from the definition of fossil fuel activity and therefore a GHG assessment is not mandatory.

Instead, to determine if a GHG assessment is needed, DEQ applies the MEPA standard of whether GHG emission impacts are potentially significant because of operation of the septic tank pumper vehicle, ARM 17.4.609(3)(d)–(e).

Based on the short operation duration, the limited amount and size of the vehicle, and the absence of any combustion‑based processes during normal operation, DEQ concludes that the operation of the proposed action would not meaningfully increase GHG emissions, and therefore no additional GHG assessment is necessary for purposes of this EA.

## Description of Alternatives

In addition to the proposed action, DEQ must also consider a "no action" alternative. The "no action" alternative would deny the approval of a land application site on the Labatte Farms property. The applicant would lack the authority to conduct the proposed activity. Any potential impacts that would result from the proposed action would not occur. The no action alternative forms the baseline from which the impacts of the proposed action can be measured.

If the applicant demonstrates compliance with all applicable rules and regulations required for approval, the “no action” alternative would not be appropriate.

Section 75-1-220(1), MCA, states, “Applicants apply for specific sites and DEQ is tasked with analyzing each site and verifying compliance with applicable law and rule. Regardless of location, if the site meets the minimum requirements established by law and rule, DEQ must consider licensure of the site.”

## Consultation

DEQ engaged in internal and external efforts to identify substantive issues and/or concerns related to the proposed project. Internal scoping consisted of internal review of the environmental assessment document by DEQ staff. External scoping efforts also included queries to the resources found in the References section at the end of this document.

## Public Involvement

After publication of this document on August 14, 2026, there will be a 30-day public comment period that ends on September 13, 2026. Adjacent landowners and other parties of interest including state and county officials will be notified of the publication of the document and the opportunity to submit comments regarding the proposed action.

## Significance of Potential Impacts and Need for Further Analysis

When determining whether the preparation of an environmental impact statement is needed, DEQ is required to consider the seven significance criteria set forth in ARM 17.4.608, which are as follows:

- The severity, duration, geographic extent, and frequency of the occurrence of the impact;

- The probability that the impact will occur if the proposed action occurs; or conversely, reasonable assurance in keeping with the potential severity of an impact that the impact will not occur;

- Growth-inducing or growth-inhibiting aspects of the impact, including the relationship or contribution of the impact to cumulative impacts – identify the parameters of the proposed action;

- The quantity and quality of each environmental resource or value that would be affected, including the uniqueness and fragility of those resources and values;

- The importance to the state and to society of each environmental resource or value that would be affected;

- Any precedent that would be set as a result of an impact of the proposed action that would commit the department to future actions with significant impacts or a decision in principle about such future actions; and

- Potential conflict with local, state, or federal laws, requirements, or formal plans.

## Conclusions and Findings

The Proposed Action would meet the requirements of the Septage Disposal and Licensure Act and associated administrative rules regulating solid waste disposal. Adherence to the septic tank pumper, solid waste, water quality, and air quality regulations and the DEQ-approved facility operation and maintenance plan would mitigate the potential for harmful releases (e.g., septage running offsite and into a water body) and impacts to human health and the environment by the  proposed action.

DEQ has prepared this EA to examine and document the effects of the  proposed action on the human environment and to determine the need to prepare an environmental impact statement through an initial evaluation and determination of the significance of impacts associated with the  proposed action. As discussed in Section 2 of this EA, DEQ has evaluated the severity, duration, geographic extent, and frequency of potential impacts to the human environment from the  proposed action. DEQ has also evaluated the probability that the impacts would occur if the  proposed action occurred. DEQ has not identified any significant environmental impacts from the  proposed action.

DEQ has not identified any growth-inducing or growth-inhibiting aspects of the  proposed action. DEQ’s approval of the proposed land application site would not set any precedent and would not commit DEQ to any future action with significant impacts, nor is it a decision in principle about any future actions that DEQ may act on. Finally, the  proposed action does not conflict with any local, state, or federal laws, requirements, or formal plans.

Based on consideration of all the criteria set forth in ARM 17.4.608, DEQ has determined that the  proposed action would not significantly affect the human environment. Therefore, an environmental assessment is the appropriate level of environmental review and preparation of an environmental impact statement is not required.

## Preparation

Environmental Assessment and Significance Determination Prepared By:

Lillian Kurzhal, DEQ Waste Management Specialist

Madeline Marchiafava, DEQ Solid Waste Hydrogeologist

Environmental Assessment Reviewed By:

Fred Collins, DEQ Solid Waste Section Supervisor

Approved By:

SIGNATURE

8/14/2026

Fred Collins, DEQ Solid Waste Section Supervisor			Date

Department of Environmental Quality

## References

Montana Tech of the University of Montana, Montana Bureau of Mines and Geology (MBMG),

Ground Water Information Center http://mbmggwic.mtech.edu/

United States Fish & Wildlife Service, Environmental Conservation Online System, 2023

ECOS: Home (fws.gov)

Montana Natural Heritage Program, 2025

http://mtnhp.org/default.asp

Montana Sage Grouse Habitat Conservation Program, 2025

http://sagegrouse.mt.gov

U.S. Department of Agriculture

https://websoilsurvey.nrcs.usda.gov/app/WebSoilSurvey.aspx

National Park Service

https://www.nps.gov/subjects/air/class1.htm

Montana Cadastral

http://svc.mt.gov/msl/mtcadastral

Fertilizer Guidelines for Montana Crops, Montana State University, 2005

https://store.msuextension.org/publications/AgandNaturalResources/EB0161.pdf

Administrative Rules of Montana

http://deq.mt.gov/Portals/112/deqadmin/dir/documents/Legal/Chapters/CH50-08.pdf

EPA Domestic Septage Regulatory Guidance, 1993

https://www.epa.gov/biosolids/domestic-septage-regulatory-guidance-guide-epa-503-rule

Google Earth, 2023

https://earth.google.com/web/search/48.26730%C2%B0,+-114.40213%C2%B0

Montana DEQ’s GIS Portal

https://gis.mtdeq.us/portal/apps/mapviewer/index.html

University of Montana

https://www.umt.edu/geosciences/documents/self-guided-geologic-field-trip-msla.pdf

USGS National Hydrography Dataset

https://www.usgs.gov/national-hydrography/national-hydrography-dataset

State Historic Preservation Office

https://mhs.mt.gov/shpo/