# Draft Standard Operating Procedures for Lock-Out Tag-Out (LO/TO)

(29 CFR 1910.147)

This standard covers the servicing and maintenance of machines and equipment in which the energization, or startup of the machines or equipment, causes a release of stored energy that could harm employees. This standard establishes minimum performance requirements for the control of such hazardous energy.

- Each piece of equipment that has the potential to injure the operator with the release of stored energy, either electrical or hydraulic, must be locked out and tagged

- There is a way to lock out almost everything in a wastewater treatment plant (WWTP)

  - Valves can be chained in place

  - Covers are available to go over handwheels that can be locked

  - Electrical panels can be locked

  - Individual breakers can be covered and padlocked

  - Starter units have a location available to insert a lock or hasp through to prevent power from being turned on

- Each person working on the piece of equipment should place their lock on the unit

- A multiple lockout hasp is used when there will be more than one person working on a piece of equipment. This item allows multiple locks to be attached to a single unit. Each person working on the unit should have their lock and tag attached to the multiple lockout hasp

- Do not cut off someone else's lock. You have to find the person whose lock is still on the unit and have them remove it

Multiple Lockout Hasp	Cover to Lockout a Handwheel

- The tag that is attached to the lock to indicated who locked out the equipment should contain the following information:

  - Name of the person who is locking out the piece of equipment

  - The date that the equipment is locked out

  - The tags are usually labeled

    - “Danger”

    - “Do Not Operate”

    - “Equipment is Locked Out”

- When all work is completed on the equipment, the lock/s and tag/s can be removed and the equipment can be placed back into service

PROCEDURE TO FOLLOW WHEN LOCKING OUT EQUIPMENT

- Have tag, lock, and locking device at the site

- Make sure the equipment is turned off

- Complete the tag with your name and the date

- Use the lockout device to make the equipment inoperable

- Attach tag and lock to the lockout device

- Perform work on the equipment

- Remove the tag and lock from the lockout device

- Check unit for proper operation

- If the equipment is working properly, put the unit back into service

- Return tag, lock, and locking device to the proper storage location